Sim7400 : A Digital Logic Circuit Simulator

Michael P. Rief

Version 1.0

SECTION 1 : INTRODUCTION

Description of Program

Sim7400 ��executes �by reading an input file that �describes �a

digital �circuit �and �then simulating this circuit.  ��The �digital

circuit must be comprised of a subset from the 7400 �family of chips

that �Sim7400 �can recognize.  �This subset is listed in appendix �A.

Output, which can be saved to a file or viewed on the screen, can be

in one of two forms.  �If the inputs to �the circuit were given as 0

or 1, �the results are the outputs of the circuit and their �values.

If �any �of the �inputs were not given a value, �(the �program �will

generate �values for these automatically) ��the �output �is a �truth

table.    �Sim7400 �allows variable names for the inputs and outputs

of the circuit.  �This improves the readability and understanding of

the �circuit. �The program will detect some input file syntax errors

along with illegal pin �usage. �It will not catch logic errors, �and

these errors will show up in the output. �It �is left up to the user

to verify and interpret the results.    �Sim7400 �can be operated in

two "modes".  The command-line mode is the fast and efficient way of

simulating �digital �circuits.  �The interactive-mode is slower �but

more �user-friendly.  ��There �are �two different modes because some

users may prefer their own text editor to create input files and may

want the speed and ease the command-line version gives them.  Others

may �not �have �an editor or may prefer �the �step-by-step �approach

available with the interactive version.  Both versions are contained

in the one executable program "sim7400.exe".

NOTE: This file may be renamed to something that is easier to type

in, such as simulate.exe" or "sim.exe".

Motivation for Program

Sim7400 ��is �not the only digital logic �simulator �available.

There �are many to choose from, �with most having more sophisticated

features.  The motivation for Sim7400 came from the size and cost of

these �other �packages.  ��It is not uncommon for �a �digital �logic

simulator �to �cost in the hundreds of dollars and require a lot �of

hard disk space and memory.  In addition, �these large and expensive

programs �have a fairly steep learning curve associated �with �them.

Sim7400, �on the other hand, is small, does not require a hard drive

or megabytes of memory, �and is easy to learn and use.  Best of all,

it �is distributed as shareware; �this means it doesn't cost a cent!

In �addition, ��the source code is distributed with �the �executable

version,  ��allowing the program to be modified to �suit �individual

needs with any ANSI-compatible C compiler.  (Please consult appendix

C for instructions on compiling the source code.)

Sim7400 �can be used by both the student taking an introductory

course in digital design and the student who has more experience �in

this �area.  ��It �is meant to be a tool that �will �verify/check �a

digital circuit design. �It will not design the circuit; �the design

of a circuit must be complete to use this package.

Scope of Manual

This manual is written assuming the reader has an understanding

of �the TTL LS 7400 �family of chips and basic digital logic design.

If the reader is not well versed in digital design, �there are �many

excellent books available on this topic.

Help Documentation

Help ��is �available �in �two �forms: ��the �distribution �file

(sim7400.arc) ��has a "help.doc" �file �in it�along with this manual

(sim7400.doc), ��and this manual has the help.doc file reproduced in

appendix �D.  Help.doc is a �condensed version of this manual and is

intended as a quick reference guide only.

SECTION 2 : SYSTEM REQUIREMENTS

Ansi.sys Driver

Sim7400 �makes extensive use of the console driver �"ansi.sys",

therefore �this driver must be installed before using this �package.

To �install �ansi.sys the "config.sys" �file on your boot disk �must

contain the line:

device=[DRIVE:]PATHNAMEansi.sys

In this line, �[DRIVE:]�is the disk drive on which the file ansi.sys

exists �and �PATHNAME �is the directory location for ansi.sys.  ��If

[DRIVE:] is omitted, �DOS assumes that ansi.sys is on the disk drive

used to boot the system.  �Notice there are no spaces on either side

of the equals sign.  An example of this syntax is:

device=C:\DOS\ansi.sys

This states that ansi.sys can be found on disk drive C, �in the  DOS

subdirectory.  Consult a DOS manual if more information is needed.

Hardware

Sim7400 �can �easily be used on systems with �only �one �floppy

drive ��and �only uses about 135K of memory. �A �monochrome �or �CGA

compatible video card is required. (Sim7400 �will work with Hercules

cards.)

Software

Although �the interactive mode of Sim7400 �will create an input

file and allow the file to be editted with respect to syntax �or pin

usage �errors, ��this package really requires the use of an �editor.

This �is �a �requirement that is easily met since text �editors �are

ubiquitous in the PC world.

SECTION 3 : INPUT FILES

Basics

Sim7400 ��executes by reading a file that describes �a �digital

circuit and then simulating this circuit, �with the results going to

the �screen �or to a file.  �Input files describe a circuit �with �a

syntax the program can understand;  any editor can be used to create

these files.  �Input files are easy to create because this syntax is

intuitive �and �free-form.��This means that text can �start �in �any

column; �the leading blank spaces are ignored. �This syntax �borrows

heavily from the actual chip numbers and pin numbers the 7400 family

of chips uses.

Input and Output Variable Names

In most digital logic circuits, inputs and outputs to gates are

given �"names" �like  X or Y.  �Sim7400 �allows much more meaningful

names.  ��Variable �names are limited in length to 160 ��characters,

however, �any combination of �characters may be �used (with the four

exceptions �noted below). �The only limit to the number of �variable

names �one �input file can have is the amount of �memory �available.

Because �Sim7400 ��is not case sensitive, �variable �names �such �as

"input_x" and "INPUT_X" are treated as the same name.

Comments

Comments are allowed; in fact, they are encouraged!  A �comment

begins �with a pound sign (#).  �Everything after the pound sign �is

considered �the comment and is ignored by the program.  �This �means

that comments �can �be placed at the end of a line or on a �line �by

themselves.  ��For �obvious reasons, �they cannot be placed �at �the

beginning �of �a line which contains data �describing �the �circuit.

Comments can be used to title files, �to describe different sections

of a circuit and to make a file generally easier to read.

Blocks

An input file can be broken into three blocks: the Input block,

the Output block, �and the Circuit Description block.  The Input and

Output  �blocks must be placed before the Circuit Description block,

however, �the order of the Input and Output blocks is not important.

The Input and Output blocks are analogous to the VAR statement in �a

Pascal program.  They declare variable names the circuit is going to

use �as inputs or outputs.  �The Circuit Description block is like a

procedure in a Pascal program: �it tells Sim7400 �how to connect the

chips that make up the circuit to be simulated.

The Input Block

In addition to declaring input variable names, �the Input block

is �used to initialize variables to a constant �value �and �to �tell

Sim7400 �how many input variables are �used �in the file.  Variables

may �be �given �the constant values one (1) �or zero (0).  ��If �any

variable �is �not given a value, ��the �program �will �automatically

generate �values �for it.  �The examples given below should �clarify

this.

NOTE: �The maximum number of variables that the program can generate

values �for �is �16; �this will produce more than �65,536 �lines �of

output! Also, if any values are generated by the program, the format

of �the output is affected. �This is discussed in greater detail �in

the section "Program Output".

The �Input �block looks exactly like a �compound �statement �in

Pascal. It begins with the reserved word Input.  �A pair of reserved

words, ��Begin �and End, �delimit the block. �Immediately after �the

reserved word Begin, �the number of input variables that are used in

the file is given.  �If a file used four input variable names, �then

after �the Begin in the Input block would come the number four �(4).

Sim7400 �does �not �distinguish �between upper-case �and �lower-case

letters.  Therefore, "INPUT" is treated exactly the same as "Input",

"input", �etc. �Although all of these are the same word to �Sim7400,

they �are �reserved words and cannot be used as �input �(or �output)

variable �names.  �If they are used, �the result will �be �a �syntax

error. �Between the reserved words Begin and End, the input variable

names �are given with either a space, �new line, �or �comma �between

them.  �If �a variable is to be given a value, �it is followed by an

equals sign �(=) �and the value.  �Some examples of Input blocks are

given below:

Example 3.1:      Input

Begin

3

input_x = 1, input_y = 0, input_z = 1

End

In this example,�all variables are initialized. �The "3" �after

the �Begin statement signals to Sim7400 �that there �are only �three

input variables used in �this �file. �Notice the use of indentation.

This �drastically �improves �readability and will most �likely �help

catch �any �syntax �errors.  �The same Input block could �have �been

written as:

Input 3 Begin input_x = 1 input_y = 0 input_z = 1 End

which shows how NOT to write Input blocks!

Example 3.2:    INPUT

BEGIN 3

Input_Bit_A     # Values will be generated

Input_Bit_B     # Values will be generated

Carry_In = 1    # This value is constant

END

This example shows a good use for comments, �how upper-case and

lower-case �are treated the same, �and the free-form of �the �syntax

(the 3 is not on a line by itself).  �The values for Input_Bit_A and

Input_Bit_B �will �automatically be �generated, �while the �variable

Carry_In has the constant value 1.

The Output Block

The �syntax of the Output block is exactly the same as that �of

the Input block.  �The only differences are the word "Output" begins

the �block �and output variables cannot be initialized to �a �value.

Another reserved word is now added to the list of forbidden variable

names: ��Output.  ��This brings the list of reserved word �to �four:

"input", "output", �"begin" and "end".  Don't forget that variations

of �case on these words are considered �the same thing,�ie: "outPUT"

cannot �be used as a variable name. �Some examples of Output �blocks

are given below:

Example 3.3:    OUTPUT

BEGIN

2 Carry_Out, Sum

END

Here �two output variables are being declared: ��Carry_Out �and

Sum.  Again the free-form is shown by the "2" being before the first

variable name declared.

Example 3.4:    output

begin 3

OUTPUT_1

OUTPUT_2

OUTPUT_3

end

While this is perfectly correct and will work, �a more readable

form would be:

OUTPUT

BEGIN 3

output_1, output_2, output_3

END

The Circuit Description Block

This is �the block that does all the work.  It is here that the

actual �chips and pin connections of the circuit �are �given.  ��The

syntax for this block is in the form TOKEN = �TOKEN, where the token

on �the �left-hand side of the equals �sign�is �assigned the current

value �of the token on the right.  �This same syntax can be given in

three other forms:

VARIABLE NAME = "7400_chip"."chip_number_in_circuit"."pin_number"

"7400_chip"."chip_number_in_circuit"."pin_number" = VARIABLE NAME

"7400_chip"."chip_number_in_circuit"."pin_number" = constant

VARIABLE NAME is the name of either an Input or Output variable

that �has been declared in the appropriate block.  ��If the variable

has not been �declared, ��a ��syntax �error will occur.  Notice that

variable �names �do �not have to be used exclusively; �constants �of

either one (1) or zero (0) can be used.  "7400_chip" �stands for the

numerical �number �of �the 7400 �chip without �the �"74Ls" ��prefix.

"chip_number_in_circuit" ��represents the number of the chip in �the

actual circuit layout.  This is because a circuit may have more than

one �of �the �same type of chip and �Sim7400 �needs to �be �able �to

distinguish �between these. �"pin_number" �is the number of the �pin

being used for the represented chip. �The syntax requires the "dots"

(.) as separaters.  To clarify this, a couple of examples are given.

Example 3.5:              86.3.2 = Input_X

"86" �represents the chip 74Ls86 �(a quad, two-input XOR chip).

".3" is used to distinguish this particular 74Ls86 from other 74Ls86

chips �in �the �circuit.  ��This �is chip number �three �(3). ��".2"

represents �pin number two (2) �of this particular chip.  ��Finally,

"Input_X"  is  the  name  of  the  variable  whose current  value is

assigned to pin 2.

Example 3.6:            Output_Y = 00.12.3

This is an example of an Output variable being assigned a value

from a chip.  Here, "00" represents the chip 74Ls00, which is a quad

two-input �NAND gate.  �The ".12" �signifies this is the 12th 74Ls00

chip in the circuit.  �The ".3" tells Sim7400 �to assign to Output_Y

the current value of pin number 3 from this chip.

It is important to know when a pin holds a valid value.  �If an

output �variable (or pin) �is assigned a value from a chip (or gate)

that does not yet have any valid inputs, �this will �cause an error.

Therefore, ��it is important to make sure all inputs to a �chip �(or

gate within a chip) ��are valid before trying to use the �output �of

that chip (or gate).

Some �examples of complete input files are given in appendix �B

along �with a "skeleton" �file that can be used as a �template �when

creating �input �files.  �As will be discussed in the �section �"The

Interactive �Mode", �Sim7400 �does have the ability to create �input

files free of syntax errors. However,  logical errors simply can not

be detected and can occur!

SECTION 4 : PROGRAM OUTPUT

Two Output Formats

The functioning of most digital logic circuits is explained �by

a truth table.  Truth tables present large amounts of information in

a �form �that is easy to read and understand.  �They �represent �the

outputs �of �the circuit for all possible �combinations �of �inputs.

When the input �to the circuit is just a single set of �input �data,

however, there is no advantage to  using a truth table.  �It �is for

this reason that Sim7400 �generates �output �depending �on the input

variables and how they are declared.

If �all �input variables are given as �constants, ��the output

format �is �not �in the form of a truth �table �(for �reasons �noted

above). �Instead, �it is simply a listing of the input variables and

their �constant values, �and the output variables with their values.

This is shown by an example.

Example 4.1:          Input

Begin

2

input_1 = 1

input_2 = 0

End

The above input block is given in a file.  Assuming the circuit

is a simple AND gate with input_1 �and input_2 �as its inputs, ��and

assuming the Output variable is output_1�(its declaration and Output

block are not shown), the results would look like:

Input Variables

---------------

input_1 = 1

input_2 = 0

Output Variables

----------------

output_1 = 0

If �any input variable is not initialized to a constant �value,

its values will be generated by the program.  This way, all possible

combinations �of �the �inputs (that were not �initialized) ��can �be

checked.  ��For example, �if the following Input block is used in �a

file:

Example 4.2:          Input

Begin

2

input_1, input_2

End

the �program �will generate values for input_1 �and input_2. ��These

values are generated in two's complement addition fashion.  �This is

shown below.

input_1  input_2

----------------

0       0

0       1

1       0

1       1

Each �line �above represents one "cycle" ��that �Sim7400 ��goes

through when simulating a circuit with these inputs.  The two inputs

are �assigned �the �values from first row, �the �entire �circuit �is

simulated and the results are output; the second row is then used as

inputs ��and the circuit is simulated again.  �This happens for �all

rows �and �the results are presented in the form of a  truth �table.

However, �if any variable is given a value in the Input block, it is

left �out �of this cyclic �algorithm �and its constant value is used

throughout�all the simulations of the circuit.  �This gives �Sim7400

great flexibility.   �This flexibility is demonstrated with �another

example.

Example 4.3:          Input

Begin

3

input_1, input_2

input_3 = 0

End

In this example, input_3 is given the value 0.  This will remain

constant throughout the simulations as explained above.  �The "input

half" of the truth table that Sim7400 will generate is:

A  B  C

--------

0  0  0

0  1  0

1  0  0

1  1  0

Only �the "input half" �can be shown since no �information �is

known  about the �circuit or its outputs.  �This �example also shows

how Sim7400 �"maps" input (and output) �variable names to letters of

the �alphabet when producing a truth table.  �In the actual �output,

these mappings are �stated �explicitly �so �there �is �no �chance of

misinterpretation of the results.   In this case, �input_1 is mapped

to A, �input_2 �is mapped to B and input_3 �is mapped to �C.  Notice

that C (input_3) does not change.

This �characteristic �of �Sim7400 ��can �be �advantageous �when

verifying circuit performance.  If a circuit has 6 inputs, �it would

not be of any value if all 6 inputs were left to be generated by the

program, since this would produce 64 lines of output!  A better idea

is �to �allow the program to generate 3 input values at a time �with

the other 3 held at a constant.  Then there would only be 8 lines of

output to verify.  If this is done often, �it speeds the "debugging"

process of circuit design considerably.

SECTION 5 : THE COMMAND LINE MODE

Sim7400 ��executes �by �reading �an input file that describes a

digital �circuit �and then simulating this circuit.  �Output can �be

saved to a file or viewed on the screen.  Sim7400 can be operated in

two "modes".  The command-line mode is the fast and efficient way of

simulating digital �circuits.  ��The interactive-mode is slower �but

more user-friendly.  ��There �are �two different modes because �some

users �may prefer to use their own text editor to create input files

and may �want the speed and ease of the command-line version. Others

may not �have an �editor or �may �prefer �the �step-by-step approach

available with the interactive version.  It is the command-line mode

that is discussed here.

The command-line �mode is the easier of the two modes �to �use,

however, �it is not as "friendly".  �When an error is encountered in

this mode, �Sim7400 �will end abruptly with a beep and a one or �two

line �error message.  �The interactive mode at least offers you �the

chance to edit the offending line and try again!

It �is �very easy to use the command-line version �of �Sim7400.

Simply enter "sim7400" followed by an input filename and an optional

output filename.  If an output filename is given, the results of the

simulation �are printed to that file.  �If �no �output �filename �is

given, �the results are shown on the screen (with scrolling for long

output).   It is then up to the user to verify the results.

If the input file does not exist, �or cannot be found using the

current �PATH setting (refer to a DOS manual about this �environment

variable), ��the �program reports this and exits.  �If a �file �that

already exists is given as the output filename, �Sim7400 will report

this and ask if this file is to be written over or appended to.  The

user also has the choice of entering a new output filename.

Below are some examples of the command-line mode.

Example 5.1:     a:> sim7400 adder.in

In �this �example �"adder.in" ��is the input file containing �a

circuit �description.  ��Because no output filename was given, ��the

simulation results are printed on the screen.

Example 5.2:     a:> sim7400 adder.in adder.out

This �time an output filename, �"adder.out", �was given.  ��All

output �will �be printed to this file (assuming it does not �already

exist) and can be viewed by the user.

SECTION 6 : THE INTERACTIVE MODE

The �advantage to using the interactive mode of Sim7400 is �the

ability to create input files easily.  This was not the prime reason

for including an interactive �mode.  �With the use of pop-up windows

and context-sensitive help, �the interactive mode makes learning and

using Sim7400 �quite easy.  �This was the prime reason �for creating

the interactive mode. �Once familiarity with the program is �gained,

the command-line mode is faster and more efficient.

To �use Sim7400 �interactively, �type in "sim7400" �at the �DOS

prompt.  The Main Menu then appears offering five choices: �Simulate

Circuit, Create Input File, �DOS Shell, �Help and Exit.  �It is then

only �a �matter �of choosing the desired �option �and �entering �the

appropriate information at any of the prompts that appear.

Basics

There �is �help available for all the menu choices of the �Main

Menu.  �Pressing F1 �while the hilight bar is on any choice will pop

up a help screen that explains what that choice will do. �Also, some

of the �windows that are used during �the �execution �of the program

have help available. �There will be a message where this is the case

stating �that F1 �is the "help" �key. �Pressing ESC at the Main Menu

will exit the program.

Main Menu

The �Main �Menu, �as stated above, �consists of �five �choices:

Simulate Circuit, �Create Input File, DOS Shell, Help and Exit.  �To

select a choice, the arrow keys are used.  �This will scroll the hi-

lighted �bar down to each choice.  �Once the desired �selection �has

been �hilighted, ��pressing �ENTER will complete the selection.  �An

alternative �way of making a selection is to press the �letter �that

corresponds to the first letter of the desired choice.  For example,

to Simulate Circuit, press 'S'; to Create Input File, press 'C'.  As

soon as the letter is pressed, ��that �choice is selected; it is not

required to press the RETURN key. This is faster than �scrolling the

hilight �bar �down �to the choice �and �hitting ENTER.  ��The "Exit"

selection on the menu quits Sim7400; �this can also be �accomplished

by pressing ESC.

Simulate Circuit

Choosing this option pops up a window that asks if the �program

output is to go to a file.  �If 'Y' is entered,�the window then asks

for the output file name to be entered.  If the file already exists,

Sim7400 �will report this and allows the options of overwriting �the

file, appending to the file, or entering in a new file name.  If 'N'

is �entered in response to the output going to a file, �this �window

will �ask for  the input file name.  �After the input and/or �output

file �names �have �been entered, ��Sim7400 ��simulates �the �circuit

described in the input file.  If any errors are encountered, a brief

error �message is displayed at the top of�the screen describing �the

error.  ��Then �a �window below this message is opened and the input

file �is scrolled up to the line that caused the error.  �It is then

possible �to �edit �this line.  �Once the offending �line �has �been

corrected, the circuit may be simulated again.

NOTE: ��It is only possible to edit the one line displayed, �if �the

stated �error �was �not �caused �by �the �indicated �line �(this �is

possible), �it may be required to exit the program and use an editor

to correct the error.  Use the "DOS Shell"  option (see below) to do

this.

Create Input File

Choosing �this option will pop up a window asking for the �name

of the �input �file that is to be created.  ��If �this �file already

exists, �Sim7400 will give the options of overwriting it or entering

a new name.  �Once a valid name is entered, �the program prompts for

the number of input variables that will be used�and then their names.

It �is possible to enter the variable names�with the constants �they

may be initialized to.  �It is not required to explicitly enter �the

reserved words "Input", �"Begin" �or "End".  These will be placed at

the correct locations in �the file automatically.  ��After the input

variables �have been entered, �the output variables are asked to �be

entered.  ��Again, �it is not required to enter  the reserved �words

"Output", "Begin" or "End".  Finally, the circuit description can be

entered.  �This is entered one line at a time, �with a blank line to

signal the end of input.

All the input is �done one line at a time, �with 75 �characters

allowed to a line.  �"Create Input File" �will also trap the illegal

use �of a reserved word for a variable name and ask for the name �to

be �reentered.  �This is a fast way to create input files, �however,

the drawbacks are that only 75 �characters are allowed to a line and

earlier �lines are not displayed on the screen.  �This is not a �big

consideration, �and this option was included for those users who �do

not �have �an �editor or prefer the interactive way of �building �an

input file.

DOS Shell

This �option is included to allow a temporary "exit" �from �the

program.  ��As �stated earlier, �an error caught �and �displayed �by

Sim7400 �might not have occurred at the indicated line.  This option

will �return to the DOS level and allow correcting of the input file

with �an editor.  �Any other program may be run when returned to the

DOS �level, ��if enough memory is available for that �program.  ��To

return to Sim7400, type "exit" �at the DOS prompt.  This will return

to the Main Menu.

Help

This option will bring �up and scroll a help document.  This is

the file "help.doc"  �It is a good idea to print the entire help.doc

file and use this as a quick reference sheet.  This help.doc file is

a �condensed �version of this manual.  �It is not intended to �be �a

complete reference manual to the program, this document is and it is

advised that this entire document be read before using the program.

Exit

This option exits the program.  The ESC key will accomplish the

same thing.

APPENDIX A : THE SIM7400 CHIP SET

The subset of the 7400 �family of logic chips that Sim7400 �can

simulate �is �listed below.  �If any chips outside of this �set �are

used, an error will occur.

Because �Sim7400 ��simulates the logic of a chip �and �not �the

electrical �characteristics of a chip, �chips �that �differ �in only

their �electrical �characteristics �are treated �exactly �the �same.

Therefore, a 74Ls00 �and a 74Ls01 �are both considered NAND gates by

Sim7400 although the 74Ls01 �has open collector outputs.  �This will

not effect the performance of a digital circuit.

74Ls00  -  Quad 2-input NAND

74Ls01  -  Quad 2-input NAND, open-collector outputs

74Ls02  -  Quad 2-input NOR

74Ls03  -  Quad 2-input NAND, open-collector outputs

74Ls04  -  Hex INVERTER

74Ls05  -  Hex INVERTER, open-collector outputs

74Ls08  -  Quad 2-input AND gate

74Ls09  -  Quad 2-input AND gate, open-collector outputs

74Ls10  -  Triple 3-input NAND

74Ls11  -  Triple 3-input AND

74Ls12  -  Triple 3-input NAND, open-collector outputs

74Ls13  -  Dual 4-input NAND, Schmitt trigger inputs

74Ls14  -  Hex INVERTER, Schmitt trigger inputs

74Ls15  -  Triple 3-input AND, open-collector outputs

74Ls20  -  Dual 4-input NAND

74Ls21  -  Dual 4-input AND

74Ls22  -  Dual 4-input NAND, open-collector outputs

74Ls26  -  Quad 2-input NAND buffer, open-collector outputs

74Ls27  -  Triple 3-input NOR

74Ls28  -  Quad 2-input NOR buffer

74Ls30  -  8-input NAND

74Ls32  -  Quad 2-input OR

74Ls33  -  Quad 2-input NOR buffer, open-collector outputs

74Ls37  -  Quad 2-input NAND buffer

74Ls38  -  Quad 2-input NAND buffer, open-collector outputs

74Ls40  -  Dual 4-input NAND buffer

74Ls86  -  Quad 2-input XOR

74Ls132 -  Quad 2-input NAND, Schmitt trigger inputs

74Ls133 -  13-input NAND

74Ls136 -  Quad 2-input XOR, open-collector outputs

APPENDIX B : EXAMPLES OF INPUT FILES

Below are two examples of input files.  These files are also in

the �distribution file sim7400.arc and can be used for demonstration

purposes.  �The files �have been reproduced here completely  as they

are �in sim7400.arc, �with slight modifications due to page lay-out.

A "skeleton" �input file, ��that can be used as a template for input

files in general, is also given.

Example B.1: adder.in

# File name: adder.in

# Purpose: Example of input file syntax

# Description of circuit: Two bit full-adder

# This is an input file that describes a two-bit full-adder circuit.

# A two-bit full-adder accepts as inputs two  bits and a carry in

# bit. It then adds the two input bits and the carry in bit to

# compute a sum bit and a carry out bit.

# This file uses inputs that are given constant values and therefore

# the output is not in the form of a truth table; the program output

#�simply lists all output variables that have been used and their

# values.

# Notice the use of indentation and blank lines.

# This greatly improves the readability of the file and will

#   help in detecting syntax and logical errors.

INPUT                   # Reserved words are all capitalized.

BEGIN                 # This is not necessary, but it does

3                   # improve readability.

input_bit_x = 1

input_bit_y = 0

carry_in = 1

END

OUTPUT

BEGIN

2

sum, carry_out

END

# To help understanding the circuit, short descriptions of

# the chips, gates and different sections used can be given.

# 74Ls86 : 2-input XOR gate

86.1.1 = input_bit_x

86.1.2 = input_bit_y

# 74Ls08 : 2-input AND gate

08.1.1 = input_bit_x

08.1.2 = input_bit_y

# XORing the carry_in bit with the

# output of the first gate in chip 86.1

86.1.4 = 86.1.3

86.1.5 = carry_in

# ANDing the carry_in bit with the

# output of the first gate in chip 08.1

08.1.4 = 86.1.3

08.1.5 = carry_in

# 74Ls32 : 2-input OR gate

32.1.1 = 08.1.6

32.1.2 = 08.1.3

# Output variable "sum" is the output of

# the second gate in chip 86.1

# Output variable "carry_out" is the output of

# the first gate in chip 32.1

sum = 86.1.6

carry_out = 32.1.3

Example B.2: nand3.in

# File name: nand3.in

# Purpose:   Example of input file syntax

# Description of circuit: 74Ls10 chip

# This input file shows how to have Sim7400 automatically

# generate input values. �

# �This is done by not giving �an �input variable a constant value

# when it is declared.  �The program �will then generate the values

# for that variable.

# This file also shows that some variables can be intitialized and

# while others don't have to be.

INPUT

BEGIN

9

in1, in2, in3               # These values will be generated.

in4, in5, in6               # These values will be generated.

in7 = 1, in8 = 1, in9 = 1   # These values stay constant.

END

OUTPUT

BEGIN

3

out1, out2, out3

END

10.1.1  = in1

10.1.2  = in2

10.1.13 = in3

out1    = 10.1.12

10.1.3 = in4

10.1.4 = in5

10.1.5 = in6

out2   = 10.1.6

10.1.9  = in7

10.1.10 = in8

10.1.11 = in9

out3    = 10.1.8

Example B.2: "Skeleton" Input File

# File name:

# Purpose:

# Description of circuit:

INPUT

BEGIN

# Replace this comment line with the number of input variables

# Replace these comment lines with the declarations of the input

# variables themselves.

END

OUTPUT

BEGIN

# Replace this comment line with the number of output variables

# Replace these comment lines with the declarations of the output

# variables themselves.

END

# This is where the circuit description goes.

# Remember to use comments liberally in input files

# for readability.

APPENDIX C : COMPILING THE SOURCE CODE

The �distribution �file, �sim7400.arc, �contains �the �complete

source �code �for �the program sim7400.  �This is included �for �the

purpose �of �modification.  ��There �are some �chips �that �are �not

simulated and a user may want to include one of these chips in their

circuit.  �The way around this problem is to write a subroutine that

simulates any chip that is needed and recompile this program.  ��The

file �that �has �the �routines �to �simulate �the �chips �is �called

"simulate.c".  ��This is where any code that simulates a chip should

be  �placed.  ��Also, ��the file "macro.h" �contains �a �macro �that

initializes �an �array with pointers to the functions that �simulate

the chips used in the program.  This must also be changed to reflect

the �added �subroutines.  �It should be obvious how to �do �this �by

looking at the existing code.  The indices to the array "chip_table"

are �the �numbers �of the chip that �are �simulated. ��For �example,

"chip_table[136]" refers to chip 74LS136.  The array is an �array of

pointers �to functions, �therefore the name of the �added �functions

come next on the line.  It would be wise to follow the syntax of the

file and give the function names meaningful names that describe �the

chip they are simulating.  A complete example would be:

chip_table[138] = 1_OF_8_DECODER;

This �intializes cell 138 �of the array chip_table to point �to

the subroutine "1_OF_8_DECODER".  �The name chosen tells at a glance

what chip is being simulated: �it is a 74LS138, �which �is �a 1 of 8

decoder/demultiplexer.

Sim7400 ��was �written and compiled with a �shareware �library,

"Tools4C" (which is included in sim7400.arc) for the window and menu

functions.  The object code must be linked with this library.  Refer

to �the specific compiler that is being used �to compile the program

on �how �to do this.  �Also, �the manual for the library �notes �the

specific include (# include) files that must be used for the library

to work properly.

For more instructions on how to compile the source code, please

read the file "readme" which is included in the self-extracting file

"source.exe".

APPENDIX D : HELP.DOC

DESCRIPTION OF PROGRAM

Sim7400 ��executes �by reading an input file that �describes �a

digital circuit and then simulating this circuit.  Output, which can

be �saved to a file or viewed on the screen, �can be in one �of �two

forms.  ��If �the inputs to the circuit were given as 0 or �1, ��the

results are the outputs of the circuit and their values.  �If any of

the inputs were not given a value, �the program will generate values

for these automatically and �the output is a truth �table. ��Sim7400

allows variable �names �for the inputs and outputs of �the �circuit.

This improves the readability and understanding of the circuit.  The

program will detect some input file syntax errors along with illegal

pin �usage. �It will not catch logic errors, �and these errors �will

show �up �in the output. �It is left up to the user �to �verify �and

interpret the results. �Sim7400 can be operated in two "modes".  The

command-line �mode �is �the fast and �efficient �way �of �simulating

digital �circuits.  �The interactive-mode is slower but �more �user-

friendly.  ��Sim7400 ��makes �extensive use of �the �console �driver

"ansi.sys". ��Therefore this driver must be installed �before �using

this �package. �To install this driver, �refer to the user's �manual

(Sim7400.doc) or a DOS manual.

INPUT FILES

Input �files �describe a circuit with a syntax the program �can

understand; ��any �editor can be used to create these �files.  ��The

program also has an option to create an input file; this is the Main

Menu choice "Create Input File".  �The syntax understood by Sim7400,

borrows �heavily �from the actual chip numbers and pin �numbers �the

7400 family of chips uses.

An input file can be broken into three blocks: the Input block,

the Output block, �and the Circuit Description block.  The Input and

Output �blocks must be placed before the Circuit Description �block,

however, �the order of the Input and Output blocks is not important.

The �Input and Output blocks are analogous to the VAR statement in a

Pascal �program.  ��They declare variable names that the circuit �is

going to use as inputs or outputs.  The Circuit Description block is

like �a �procedure in a Pascal program: �it tells �Sim7400 ��how �to

connect the chips that make up the circuit to be simulated.

The Input Block

In addition to declaring input variable names, �the Input block

is �used �to �initialize variables to a constant value and �to �tell

Sim7400 �how many input variables are used in the file.  ��Variables

may �be �given �the constant values one (1) �or zero (0).  ��If �any

variable �is �not �given a value, �the �program �will �automatically

generate values for it.

NOTE: The maximum number of variables that the program can generate

values for is 16; this will produce more than 65,536 lines of

output!

The Input block begins with the reserved word "Input".  A �pair

of reserved words, "Begin" and "End", delimit the block. Immediately

after �the reserved word Begin, �the number of input variables �that

are used in the file is given. �Sim7400 does not distinguish between

upper-case and lower-case letters.  �Therefore, �"INPUT" �is treated

exactly the same �as "Input", �input", �etc. �Notice that these four

words  �are reserved words and therefore cannot be used as input (or

output) �variable names.  �If they are, �the result will be a syntax

error. Between the reserved words Begin and End, �the input variable

names �are �given with either a space, �new line, �or comma �between

them.  �If a variable is to be given a value, �it is followed by �an

equals sign (=) �and the value.  �For some examples of Input blocks,

refer to the manual Sim7400.doc.

The Output Block

The syntax �of the Output block is exactly the �same as that of

the Input block.  �The only differences are the word "Output" begins

the �block �and output variables cannot be initialized to �a �value.

Another reserved word is now added to the list of forbidden variable

names: ��Output.  ��This brings the list of reserved word �to �four:

input", �"output", "begin" and "end".  �Don't forget that variations

of �case on these words are considered the same thing,ie: ��"outPUT"

cannot �be �used as a variable name. �Again, �for some �examples �of

Output blocks, refer to the manual Sim700.doc.

The Circuit Description Block

It �is �here that the actual chips and pin connections �of �the

circuit are given.  The syntax for this block is in the form TOKEN =

TOKEN, �where the token on the left-hand side of the equals sign �is

assigned �the �current value of the token on the right.  �This �same

syntax in another form is:

VARIABLE NAME =  7400_chip_number.chip_number_in_circuit.pin_number

OR

7400_chip_number.chip_number_in_circuit.pin_number = VARIABLE NAME

OR

VARIABLE NAME = constant

OR

7400_chip_number.chip_number_in_circuit.pin_number = constant

VARIABLE NAME is the name of either an Input or Output variable

that �has been declared in the appropriate block.  �If the �variable

has ���not ��been ��declared, ���a ���syntax ��error ��will ��occur.

"7400_chip_number" stands for the numerical number of the 7400 �chip

without the "74Ls" �prefix.  "chip_number_in_circuit" represents the

number of the chip in the actual circuit layout.  �This is because a

circuit �may have more than one of the same type of chip and Sim7400

needs to be able to distinguish between these.  "pin_number" �is the

number �of the pin being used for the represented chip. �Because �an

equals sign (=) is next, the value of the token to come next will be

used for the pin of the chip just described.  �Notice that �variable

names �do not have to be used, �constants of either one (1) �or zero

(0) �can be used.  �It is important to know when a pin holds a valid

value.  �If an output variable (or pin) �is assigned a value from �a

chip �(or gate) �that does not yet have any valid inputs, �this will

cause an error.  Therefore, �it is important to make sure all inputs

to a chip (or gate within a chip) are valid before trying to use the

output �of �that �chip (or gate).  �Once again, ��some �examples �of

complete �input �files �are �given �in �appendix �B �in �the �manual

Sim7400.doc, ��along with a "skeleton" �file that can be used �as �a

template when  creating input files.

PROGRAM OUTPUT

The functioning of most �digital logic circuits is explained by

a truth table.  Truth tables present large amounts of information in

a �form �that is easy to read and understand.  �They �represent �the

outputs �of �the circuit for all possible �combinations �of �inputs.

When �the �input to the circuit is just a single set of input �data,

however, �there is no advantage to  using a truth table.  �It is for

this �reason �that Sim7400 �generates output depending on the �input

variables �and �how they are declared.  �If all input variables �are

given as constants, �the output format is not in the form of a truth

table (for reasons noted above). �Instead, it is simply a listing of

the �input �variables �and their constant values, ��and �the �output

variables ��and �their �values.  ��If �any �input �variable �is �not

initialized to a constant value, its values will be generated by the

program.  This way, all possible combinations of these inputs can be

used to check the operation of the circuit. This gives Sim7400 great

flexibility.