DESCRIPTION OF PROGRAM

Sim7400 ��executes �by �reading an input file �that �describes �a

digital circuit and then simulating this circuit.  �Output, �which can

be �saved �to �a file or viewed on the screen, �can be in one �of �two

forms.  If the inputs to the circuit were given as 0 or 1, the results

are the outputs of the circuit and their values.  If any of the inputs

were �not �given a value, �the program will generate values for �these

automatically and the output is a truth table. Sim7400 allows variable

names �for the inputs and outputs of the circuit. �This �improves �the

readability and understanding of the circuit.  The program will detect

some �input file syntax errors along with illegal pin usage. �It �will

not catch logic errors, �and these errors will show up in the �output.

It �is �left �up �to the user to verify �and �interpret �the �results.

Sim7400 �can be operated in two "modes".  The command-line mode is the

fast ��and �efficient �way �of �simulating �digital �circuits.  ���The

interactive-mode �is �slower but more user-friendly.  �Sim7400 ��makes

extensive use of the console driver "ansi.sys". �Therefore this driver

must be installed before using this package. �To install this �driver,

refer to the user's manual (Sim7400.doc) or a DOS manual.

INPUT FILES

Input �files �describe a circuit with a syntax �the �program �can

understand; any editor can be used to create these files.  The program

also �has �an option to create an input file; �this is the �Main �Menu

choice "Create Input File".  The syntax understood by Sim7400, borrows

heavily from the actual chip numbers and pin numbers the 7400 ��family

of chips uses.

An �input file can be broken into three blocks: �the Input block,

the Output block, �and the Circuit Description block.  �The Input �and

Output �blocks �must be placed before the Circuit �Description �block,

however, ��the order of the Input and Output blocks is not �important.

The �Input and Output blocks are analogous to the VAR statement �in �a

Pascal program.  They declare variable names that the circuit is going

to use as inputs or outputs.  �The Circuit Description block is like a

procedure �in a Pascal program: �it tells Sim7400 �how to connect �the

chips that make up the circuit to be simulated.

The Input Block

In addition to declaring input variable names, the Input block is

used �to initialize variables to a constant value and to tell �Sim7400

how many input variables are used in the file.  Variables may be given

the constant values one (1) or zero (0).  If any variable is not given

a value, the program will automatically generate values for it.

NOTE: The maximum number of variables that the program can generate

values for is 16; this will produce more than 65,536 lines of

output!

The Input block begins with the reserved word "Input".  A �pair of

reserved �words, �"Begin" �and "End", �delimit the block. �Immediately

after the reserved word Begin, �the number of input variables that are

used in the file is given. Sim7400 does not distinguish between upper-

case and lower-case letters.  �Therefore, �"INPUT" �is treated exactly

the same as "Input", �input", �etc. �Notice that these four words  are

reserved �words �and �therefore cannot be used as �input �(or �output)

variable names.  ��If �they are, the result will be �a �syntax �error.

Between the reserved words Begin and End, the input variable names are

given �with either a space, �new line, �or comma between them.  �If �a

variable is to be given a value, �it is followed by an equals sign (=)

and the value.  For some examples of Input blocks, refer to the manual

Sim7400.doc.

The Output Block

The syntax of the Output block is exactly the same as that of the

Input �block.  �The only differences are the word "Output" �begins the

block �and output variables cannot be initialized to a value. �Another

reserved �word is now added to the list of forbidden �variable �names:

Output.  ��This �brings �the list of reserved word to �four: ��input",

"output", "begin" and "end".  �Don't forget that variations of case on

these words are considered the same thing,ie: �"outPUT" cannot be used

as a variable name. �Again, for some examples of Output blocks, �refer

to the manual Sim700.doc.

The Circuit Description Block

It �is �here �that the actual chips and pin �connections �of �the

circuit are given.  �The syntax for this block is in the form TOKEN �=

TOKEN, ��where the token on the left-hand side of the equals �sign �is

assigned �the �current value of the token on the �right.  ��This �same

syntax in another form is:

VARIABLE NAME =  7400_chip_number.chip_number_in_circuit.pin_number

OR

7400_chip_number.chip_number_in_circuit.pin_number = VARIABLE NAME

OR

VARIABLE NAME = constant

OR

7400_chip_number.chip_number_in_circuit.pin_number = constant

VARIABLE �NAME is the name of either an Input or Output �variable

that has been declared in the appropriate block.  �If the variable has

not �been declared, �a �syntax error will �occur.  �"7400_chip_number"

stands �for the numerical number of the 7400 �chip without the �"74Ls"

prefix.  "chip_number_in_circuit" represents the number of the chip in

the �actual circuit layout.  �This is because a circuit may have �more

than �one �of the same type of chip and Sim7400 �needs to be �able �to

distinguish �between these.  �"pin_number" �is the number of �the �pin

being �used for the represented chip. �Because an equals sign (=) ��is

next, �the value of the token to come next will be used for the pin of

the chip just described.  Notice that variable names do not have to be

used, ��constants of either one (1) �or zero (0) �can be used.  �It is

important �to �know �when a pin holds a valid value.  ��If �an �output

variable (or pin) �is assigned a value from a chip (or gate) that does

not yet have any valid inputs, �this will cause an error.  �Therefore,

it �is important to make sure all inputs to a chip (or gate �within �a

chip) ��are �valid before trying to use the output of �that �chip �(or

gate).  Once again, some examples of complete input files are given in

appendix �B in the manual Sim7400.doc, �along with a "skeleton" ��file

that can be used as a template when  creating input files.

PROGRAM OUTPUT

The functioning of most digital logic circuits is explained by �a

truth table.  �Truth tables present large amounts of information in �a

form that is easy to read and understand.  �They represent the outputs

of �the �circuit for all possible combinations of inputs.  ��When �the

input �to the circuit is just a single set of �input �data, ��however,

there is no advantage to  using a truth table.  �It is for this reason

that Sim7400 generates output depending on the input variables and how

they are declared.  If all input variables are given as constants, the

output �format is not in the form of a truth table (for reasons �noted

above). ��Instead, �it is simply a listing of the input variables �and

their constant values, and the output variables and their values.  �If

any input variable is not initialized to a constant value, �its values

will be generated by the program.  This way, all possible combinations

of �these �inputs can be used to check the operation of �the �circuit.

This gives Sim7400 great flexibility.