CHAPTER ONE

INTRODUCTION

Calculations of network sensitivities can become extremely cumbersome

for the network designer.  The computer program "SENS"  provides  the

user with computer-aided analysis of network sensitivies and provides

the  network  designer with the sensitivities discussed below as well

as  graphing  utilities  that   provide   plots   of   the   function

sensitivities.

1.1  DEFINITION OF SENSITIVITY

In the equation

y    dy x     dy/y     d(ln y)

S  =  -- -  =  ----  =  -------              (1.1)

x    dx y     dx/x     d(ln x)

the letter "S" is being used to denote sensitivity,  the  superscript

character  "y"  is being used to denote the performance charateristic

to be evaluated, and the subscript letter "x" is being used to denote

the network element which is  being  changed.  Equation  1.1  defines

relative  sensitivity.  Perhaps  the  most  useful  way  to  look  at

sensitivity is in the following equation.

dy    y  dx

-- = S   --                        (1.2)

y    x   x

Equation  1.2  shows  the  relative   change   in   the   performance

characteristic,  brought  about  by the change in x,  is equal to the

sensitivity of y with respect to x times the relative  change  in  x.

This  shows that a lower sensitivity value is desired because it will

mitigate the change in the performance characteristic.

There are several choices for the  performance  characteristic.  SENS

performs  five (5) types of sensitivity analysis;  coefficient,  root

(normalized and unnormalized),  Q,  Wn,  and function (or  classical)

sensitivity.

1.2  COEFFICIENT SENSITIVITY

Generally,  a  network  function  N(s)  is  a  ratio of numerator and

denominator polynomials in the form

A(s)    A0 + A1*s + A2*s^2 + ... + Am*s^m

N(s) =  ---- = -----------------------------------        (1.3)

B(s)    B0 + B1*s + B2*s^2 + ... + Bn*s^n

where m is the degree of the numerator polynomial and n is the degree

of the denominator polynomial and the coefficients Ai and Bi are real

numbers and  are  functions  of  the  network  elements.  Coefficient

sensitivities are defined as

Ai    dAi   x        Bi    dBi   x

S   = ----- ---      S   = ----- ---           (1.4)

x     dx    Ai       x     dx    Bi

From  equation  1.4  it  can be seen the coefficient sensitivity will

depend upon how the network polynomials A(s) and  B(s)  are  defined.

For this reason, and for uniqueness, we shall assume that the highest

degree  coefficient  of  the  denominator  polynomial has been set to

unity.  It is up to the user to set the coefficient to unity.

1.3  ROOT SENSITIVITY

1.3.1  Unnormalized Root Sensitivity

The computation of root sensitivity is an important component of  the

SENS  program  because  roots  usually  play  a  major  factor in the

determination    of    the    networks    properties.    Unnormalized

(semirelative) root sensitivities may be defined as

Pi   dPi              Zi   dZi

US  = -----           US  = -----            (1.5)

x    dx/x             x    dx/x

where Pi are the poles (roots of the  denominator)  and  Zi  are  the

zeros  (roots  of  the  numerator)  of  the  network  function  N(s).

Fortunately,  the computer does not  have  to  use  equation  1.5  to

compute  unnormalized  root  sensitivities.  The  following  equation

shows that the unnormalized root sensitivity can be computed strictly

from use of the polynomial and its coefficient sensitivities.  It can

be seen from equation 1.5 that the root sensitivity for the numerator

or denominator  polynomial  is  the  same.  Thus,  in  the  following

equation,  we  will  let  the  polynomial  C(s)  represent either the

numerator or the denominator polynomial and  Ri  will  represent  the

zero  or pole.  The symbol "#" will represent epsilon in its capacity

as a summation symbol.

N         Ci

-# Ci s^i S    |

Ri  i=0        x   |

US  = -------------- |                    (1.6)

x       dC(s)      |

-------     |

ds       | s = Ri

where  x  is  an  element  in  the  network  and  Ci  represents  the

coefficents  of the polynomial C(s).  The only undefined component of

equation 1.6 is the denominator, C'(s),  which can be represent using

the following equation.

N

C'(s) = # i Ci s^(i-1)                    (1.7)

i=1

1.3.2  Normalized Root Sensitivity

To obtain normalized root sensitivity, the equation

Ri   Ki/dKi     dWi/Wi

NS  =  ------  + j------                 (1.8)

x     dx/x       dx/x

can be used where Ri = Ki + jWi.  Using equations in 1.5 equation 1.7

can be maninpulated to yield

Ri            Ri             Ri

NS  = 1/Ki Re US   + j/Wi Im US              (1.9)

x             x              x

Therefore both the unnormalized and normalized root sensitivities may

be  computed  using  manipulations  of  the numerator and denominator

polynomials,  and computations for both zeroes  and  poles  has  been

shown to be identical.

1.4  Q AND Wn SENSITIVITIES

Here,  Q  is  the  quality  factor,  Wn/BW  where  BW  stands for the

bandwidth defined as the difference between the frequencies at  which

the network function magnitude is 3dB down from its peak magnitude at

the  resonant  frequency,  Wn.  The  program "SENS" computes Q and Wn

sensitivities for second and third  degree  functions  only.  In  the

case  of  a  third degree function,  SENS computes the g sensitivity,

where (s + g) is a factor in the denominator polynomial.

A(s)    A1 + sA2 + s^2A3

N(s) = ---- = --------------------           (1.10)

B(s)   s^2 + sWn/Q + Wn^2

A(s)     A0 + sA1 + s^2A2 + s^3A3

N(s) = ---- = -----------------------------       (1.11)

B(s)    (s + g)(s^2 + sWn/Q + Wn^2)

The Q, Wn, and g sensitivities are defined as

Q    dQ/Q

S  = ------                       (1.12)

x    dx/x

Wn   dWn/Wn

S  = --------                      (1.13)

x     dx/x

g    dg/g

S  = ------                       (1.14)

x    dx/x

1.4.1  Second Order Function Sensitivities

In the second order case, the denominator B(s) can be expressed as

s^2 + sWn/Q + Wn^2  =  s^2 + sB1 + B0          (1.15)

assuming B2  is  unity.  From  1.15  an  expression  for  Q  and  the

sensitivity for Q can be derived.

Q        B0     B1

Q = SQRT(B0)/B1      S =  0.5 S   -  S         (1.16)

x        x      x

Likewise,  the an expression for Wn and the sensitivity for Wn can be

shown as

Wn       B0

Wn = SQRT(B0)        S  = 0.5 S             (1.17)

x        x

Thus  it  can  be  seen  from  equations  1.16  and  1.17  that   the

sensitivities  for  the  second degree denominator can be found using

coefficient sensitivities.

1.4.2  Third Order Function Sensitivities

SENS computes the sensitivities for third degree functions by solving

the set of simultaneous equations shown below.

B0   2Wn^2 B3   Wn     gWn^2B3   g

S  = ---------- S    + --------- S

x       B0      x         B0     x

(1.18)

B1   2WN^2 + gWn   B3   Wn     gWn   B3   Q      gWn   B3   g

S  = ------------- ---- S   -  ----- ---- S   +  ----- ---- S

x         Q        B1   x       Q    B1   x       Q    B1   x

B2   Wn   B3    Wn     Wn   B3   Q        B3   g

S  = ---- ----- S   -  ---- ---- S   +  g ---- S

x     Q   B2    x       Q   B2   x        B2   x

The equations in 1.18 can easily be solved for the  sensitivities  of

Q,  Wn,  and  g  using  a  procedure that solves simultaneous sets of

equations.  Once again,  note that the  equations  in  1.18  use  the

coefficient sensitivities.

1.5  FUNCTION SENSITIVITY

Function  sensitivity,  which  is  the  sensitivity  of  the  network

function with respect to a network element,  is usually a function of

the complex frequency variable "jw".  Therefore,  SENS provides plots

of the function sensitivity for a given frequency range.

As shown in 1.3,  the  network  function  N(s)  is  a  ratio  of  two

polynomials.  Therefore the following equation may be derived.

N(s)    dN(s)    x             A'(s)     B'(s)

S     = ------- ------  =  x [ ------- - ------- ]   (1.19)

x         dx    N(s)           A(s)       B(s)

In  equation  1.19, A'(s) and B'(s) is the derivative with respect to

x.  So from equation 1.19 we find that

M     Ai          N     Bi

# Ai S  s^i   -   # Bi S   s^i

N(s)   i=0    x          i=0    x

S    = ---------------   ---------------        (1.20)

x          A(s)              B(s)

Once again, coefficient sensitivity is used in the computation of the

desired sensitivity.

In terms of plots, we will separate the sensitivity equation into its

real  and  imaginary  components.  The  following series of equations

shows the relationship between function sensitivity and its real  and

imaginary components.

N(s)    |N(s)|                 Arg(N(s))

S     = S       +  j Arg(N(s)) S

x       x                      x

N(s)   |N(s)|         N(s)             Arg(N(s))

Re S    = S           Im S    = Arg(N(s)) S

x      x              x                x

N(s)        N(s)         N(s)

S     =  Re S    +  j Im S

x           x            x

In  chapter 2 it will be discussed exactly how SENS separates the two

components of function sensitivity.

CHAPTER 3

USER'S GUIDE

3.1  THE INPUT FILE

The sections in 3.1 cover the structure of the input  file.  At  this

point we will point out that upper and lower case letters can be used

interchangeably except when identifying elements.  In the declaration

headers  SENS  will  view  upper  and lower case letters as the same.

Examples of input files are given in Appendix B along with the output

they generate.  It is suggested the user look at  these  examples  to

get  a  'feeling' for the structure of the input file and use Section

3.1 as a reference when questions  arise.  There  is  also  a  sample

input file in Table 3.1.

3.1.1  Title of Input File

The first part to the input file is the  title  of  the  input  file.

SENS will take the first non-blank line as the title.  This means the

first  line  in the file does not have to be the title.  The title is

stored and is echoed at beginning of each sensitivity  analysis.  The

maximum length of the title, indeed any line in the input file, is 79

characters.

3.1.2  The Elements - E

After SENS reads the title, it will search the remaining lines in the

input file for  the  elements  declaration.  A  line  beginning  with

either an 'e' or a capital 'E' indicates that elements are to follow.

For  readability,  it  is suggested that the whole word 'Elements' or

'elements' be spelled out.

Following the 'elements' declaration are  the  element  literals  and

their  values.  Typical  element literals would be 'R1',  'c2',  etc.

The user can use either upper or lower  case  letters,  but  must  be

consistent when using the element literals in the  network  function.

'C2'  is not the same as 'c2' to SENS.  Note that only two characters

can be used to compose the literal.

The  user  has  the  option  of  following the element literal with a

reciprocal literal.  The  reciprocal  literal  can  be  used  in  the

network  function  to  indicate the reciprocal value is desired.  The

reciprocal literal  can  be  placed  immediately  after  the  element

literal or there can be spaces between the two literals.

To  separate  the  elements  value  from  the  element  or reciprocal

literals,  the user must place a comma or a colon between the literal

and  the  value.  The value expected is a real number,  but SENS will

accept an integer.  The value can also  be  expressed  in  scientific

notation.

There  can  be  no blank lines between 'Elements' header and the last

element in the circuit.  Also,  since SENS is looking for a  'CO'  as

the  declaration  of the constants to follow,  an element name cannot

have this combination as a literal.

An  example  of  the  syntax of element literals can be seen in Table

3.1.

_____________________________________________________________________

Title ....

Elements

r1 g1: 1

r2g2 , 2.01

C2    , 1.0E-9

l4:3.7

cOnstants

01 : 1

00,0.00

ab    ,1.E7

Nu

01,, g1C2,,++

l4r1,.

DEnom

01,, l4g2 r1C2,, 01,.

$

FNsen

Lo Rad 100

M :100 2

Phase,10 30.

Im r1 , 10 4.0

Re r1: 50 3

$

scientific

functionSen   r1

Linear Radians 50

ReSen r1 : 50 0.5

$

$

Table 3.1 - Example of input file with

l4r1*s^2 + g1C2*s + 1

N(s) = ------------------------

s^2 + (l4g2+r1C2)s + 1

Note - N(s) is for no particular network.

_____________________________________________________________________

3.1.3  The Constants - CO

The syntax for  entering  constants  is  exactly  like  that  of  the

elements.  A  line beginning with the character combinations of 'CO',

'Co',  or 'co' is the constants declaration.  An example can be  seen

in  Table  3.1.  The  blank line above the 'Constants' and 'Elements'

declarations is optional.  It is put in for readability.  Blank lines

are not allowed between  the  'Constant'  declaration  and  the  last

constant.  If  there  are  no constants in the network function,  the

'Constant' declaration still must be put in,  but followed by a blank

line.  Since  SENS is looking for an 'n' or an 'N' as the declaration

for the numerator of the network function,  any constant declared  in

the input file cannot have these characters as the first character in

the constant literal.

3.1.4  The Numerator and Denominator - N, D

The numerator declaration must follow the constants as in the example

given in Table 3.1.  A line beginning with an 'N' or an 'n' qualifies

as a numerator declaration, although it is recommended that numerator

be  spelled  out for readability.  On the lines immediately following

the declaration, the expression for the numerator must be entered.

Following the expression for the numerator, SENS looks for 'd' or 'D'

as the first character in the following  lines.  Once  again,  it  is

recommended, though not required, that 'Denominator' be spelled out.

The  syntax  rules for both the numerator and denominator expressions

are  identical,  with  an  example  in  Table  3.1.  The  denominator

declaration  and  its  expression must follow those of the numerator.

Table 3.2  gives  a  summary  of  rules  to  use  in  creating  these

expressions.

_____________________________________________________________________

|

Rule  |

Number |                      Explanation

_______|_____________________________________________________________

-------|-------------------------------------------------------------

|

1.  |   Any of the literal elements and constants must be declared

|   in the elements and constants area.

|

2.  |   The declaration  for  either numerator or denominator must

|   be followed immediately with the appropriate expression on

|   the next line.

|

3.  |   The s in the original expression must be replaced by ,, in

|   the  SENS  expression, and ++ must come after the complete

|   literal character as declared in rule 1.  A ++  can  split

|   the  expression  onto  separate  lines,  but  cannot  occur

|   within a coefficient summand.

|

4.  |   The  network   function   expression  must  be  listed  in

|   ascending powers of s, starting with s^0.

|

5.  |   All coefficents of s must appear in the  SENS  expression.

|   For  s^2 + 1, the SENS form is  01,, 00,, 01,.

|

6.  |   SENS computes the proper sign for fractions.

|

7.  |   All fractions must be in reduced literal form, i.e., if  a

|   coefficient  has  the  standard  form of R1/R1L1, then the

|   SENS form is 1/L1.

|

8.  |   Only  one  occurence  of  a literal element or constant is

|   allowed per coefficient summand.

|

9.  |   SENS cannot compute a fraction in which the denominator is

|   a summation.  I.e., the equation 1/[ s^2 + 1/(L1+C1)] will

|   have to be changed to (L1 + C1)/[ s^2(L1+C1) + 1 ].

|

10.  |   All  expression  of  the  type  +A/BC +C  will  be read as

|   (+A/BC) +C.

|

11.  |   All -'s are preceded by a blank or a slash with the latter

|   indicating division.

|

12.  |   All (++ ,, ,.) are NOT preceded by blanks.

|

13.  |   All ,,'s can only be followd by ++ or blank.

=====================================================================

Table 3.2  Network function conversion rules.

3.1.5  The Special Character '$'

3.1.5.1  Separating Function Sensitivity Parameters

In the input file, once SENS reads a line that begins with a '$',  it

knows that the next line will either be another '$', or the beginning

of  parameters for function sensitivity plots.  These parameters will

be covered in more detail in Section 3.1.6.  There can  be  no  blank

lines  any where after the first encountered '$'.  All the parameters

listed for function sensitivity must be immediately followed by a '$'

line.   The  input  file  may  have  as  many  sections  of  function

sensitivity  parameters as the user desires,  with each section being

separated by '$' lines as shown in table 3.1.

3.1.5.2  Indicating End of Data

In the input file,  two consecutive lines that  begin  with  the  '$'

character  indicates the end of the data in the file.  Anything after

these two lines  will  be  ignored  by  SENS.  Table  3.1  shows  two

sections  of  function  sensitivity parameters separated by '$' lines

before the input data is terminated with  the  consecutive  lines  of

'$'s.

3.1.6  Function Sensitivity Parameters

This part of the input file  allows  the  user  to  define  what  the

parameters for the graphs of function sensitivity with respect to the

elements  will  be.  The  user  will  specify  parameters such as the

number of plotting points,  the maximum ordinate on  the  graph,  the

scaling of the plot, etc.  It should be stressed that there can be no

blank  lines  within  the  function  sensitivity  section.  SENS also

provides plots of the magnitude  and  phase  characteristics  of  the

network function.

3.1.6.1  Scientific Notation Declaration - S - Optional

SENS  provides  a  listing  of  the data values for the magnitude and

phase  characteristics of the network function.  The user can request

that these values be expressed in scientific  notation  by  following

the  '$'  line with an 'S' ( or 's').  Otherwise these values will be

expressed in non-exponential form.

3.1.6.2  The Function Sensitivity Declaration - F

After the optional scientific declaration,  SENS will look for a 'F'.

This  is the function sensitivity declaration.  On this same line the

user as the option of restricting the function senstivity to a single

element,  as in the second function sensitivity section in Table 3.1.

The  must  be  at least one space before the element literal.  If the

user specifies an element,  that  means  that  only  plots  for  this

element will be available (along with the magnitude and phase plots).

If  the  user  does  not  specify  a  single element,  then SENS will

automatically fill in any unspecified maximum ordinates  and  scaling

factors  for an element using the specified values of the last listed

element.

3.1.6.3  Defining the Abscissa - LO, LI - R, H

The next line after the function sensitivity declaration is  used  to

define  the  scaling  of  the  frequency variable.  Here the user can

specify  logarithmic  or  linear  spacing  (with  an  'LO'  or   'LI'

declaration,  respectively), and specify if the frequency is given in

radians or hertz (with an  'R'  or  'H'  declaration,  respectively).

There  must  be  at  least  one blank space in between the log/linear

declaration and the rad/hertz declaration.  On the same line as these

two declarations must an integer indicating the number of  points  to

be  plotted  (up  to  100).  The  number  of  plotting points must be

separated from the rad/hertz declaration by a comma or  colon.  These

parameters  will apply to every graph within this particular function

sensitivity section.

3.1.6.4  The Starting and Ending Frequencies

The  next  line indicate the starting and ending frequency,  with the

units being as declared above.  The starting frequency may NOT have a

value of zero - usually a .1 or .001 will yield the desired result on

the graph.  SENS looks for two real numbers on this  line,  separated

by  a least on blank,  with the starting frequency being the first of

the two.

3.1.6.5  Magnitude and Phase Declarations - M, P

In this section the user specifies the maximum ordinate  and  scaling

to  be  used  for  the  magnitude and phase plots.  The magnitude and

phase declarations are indicated  by  a  'M'  or  'P',  respectively.

These  declarations  must  be  on  separate  lines.  Following  these

declarations must be a comma or colon,  followed by an integer  value

(for the maximum ordinate) and a real value (for the scaling).

3.1.6.6  Real and Imaginary Sensitivity Declarations - R, I

In  this  section the user specifies the maximum ordinate and scaling

to be used for the function sensitivity plots.  The declarations used

here are 'R' and  'I'  for  real  and  imaginary  sensitivity  plots,

respectively.  Following these declarations is at least one blank, an

element  literal,  a comma or a colon,  and then the maximum ordinate

and scaling values.  If the parameters for all the elements  are  not

specified and a single element as not been  declared  (see  3.1.6.2),

then  SENS  will assign parameters to the unspecified elements in the

following fashion.  The last parameters read in with respect to  real

sensitivity   will   be  transferred  to  the  remaining  unspecified

elements' real sensitivity parameters,  and the last parameters  read

in  with  respect to imaginary sensitivity will be transferred to the

remaining unspecified elements' imaginary sensitivity.

3.1.7  Summary

This concludes the documentation on the  input  file.  The  following

key rules should be kept in mind:

1:  The  order  declarations  is  fixed.  The  title comes first,

followed by elements,  constants,  numerator,  denominator,  and  any

function sensitivity parameter sections.

2: Within each section there can be no blank lines.

3:  Upper  and  lower  case  can  be  used interchangeably in the

declarations, but not in the element literals.

4: An integer value can not have a decimal point in it.

3.2  STARTING SENS

3.2.1  Versions of SENS

There are three versions of SENS - SENSPC, SENSAT, and SENSASM.  Each

refers to the type of graphics  routines  used  within  SENS.  SENSPC

gives  320x200  resolution  graphs,  usable  on most IBM-compatibles.

SENSAT and SENSASM deliver the high resolution (640x400) graphs, with

the SENSASM package using assembly routines to produce its  graphics.

Unfortunately,  a  SENSASM.COM file cannot be used since the external

HW.COM file will not integrate correctly.  Therefore, to use SENSASM,

it must be run after being compiled in the TURBO PASCAL  environment.

The  specific plot files associated with SENSASM are PLOT5ASM.PAS and

HW.COM.

3.2.2  Initial Options

When SENS first starts up,  it will ask for the  name  of  the  input

file.  Enter  the complete name of the file.  After SENS verifies the

files exists,  the user will then have a choice in  selecting  either

alphanumeric  or  graphics  plotting.  Alphanumeric  plotting  can be

useful if the user wants all the output printed to the printer  or  a

file.  A  graphics plot shows up on the screen and a screen-dump must

be performed to have the plot transferred to the printer.  Note - the

graphics.com file must be  run  first  to  prepare  the  printer  for

graphic input.  This is usually included on one of the DOS diskettes.

After the user selects a type of graphing routine,  the user then has

the options of having the output directed to the screen, the printer,

or saved to a file.  If the user picks either  the  printer  or  file

options,  then SENS will automatically run through the input file and

generate all output giving no further prompts  to  the  user.  It  is

with  the  screen  option  that  allows the user to interact with the

program while it is running.  If the user picks  the  screen  option,

SENS  will  ask if the user wishes the data from the input file to be

echoed.  If so,  SENS will list the elements and constants and  their

values,  and the expressions for the numerator  and  denominator.  If

the user does not wish to see the echoed data,  SENS will go directly

to the main SENSITIVITY OPTIONS MENU,  which is covered in  the  next

section.

3.3  THE SENSITIVITY OPTIONS MENU

This section of SENS prompts the user for  what  type  of  data  they

would like to see as well as some modification routines.  Each option

is discussed below.

3.3.1  Display Coefficients Option

This  option  merely displays the numerical value of the coefficients

of the numerator and denominator polynomials.

3.3.2  Display Roots Option

This option display the poles and zeros of the network function - the

roots of the denominator and numerator polynomials respectively.

3.3.3  Display Coefficient Sensitivity Option

This option computes and displays the coefficient  sensitivities  for

the  coefficients  of  both the numerator and denominator polynomials

with respect to each element.

3.3.4  Display Root Sensitivity Option

This option  computes  and  generates  the  root  sensitivities  with

respect  to  each  element.  The listing will include only one of any

complex conjugate roots.

3.3.5  Display Q and Wn Sensitivities Option

If the denominator is a second or third degree polynomial,  SENS will

display the Q and Wn sensitivities for this option.

3.3.6  Read in Next Section of Function Sensitivity Parameters Option

SENS  automatically  reads  in  the FIRST set of function sensitivity

parameters.  This option applies if the user has done all he  or  she

wishes  to  do  with  the current parameters and wants to read in the

next section of parameters,  provided they exist.  If there  are  not

any  more  parameters  and  the  user accidently keys this option,  a

warning  will  appear  on  the  screen,  but  the  program  will  not

terminate.  Remember, SENS automatically reads in the first set.

3.3.7  Display Function Sensitivities Option

When the user selects this option,  he or she will have the option of

viewing a listing of the data values  for  the  magnitude  and  phase

characteristics of the network function.  When SENS is done computing

the  function  sensitivity  values,  another  menu will appear on the

screen,  the FUNCTION SENSITIVITY MENU.  This menu allows the user to

select  options  with regard to creating and modifying graphs.  It is

recommended that if the user chooses the screen  option,  he  or  she

should also select the graphics option, for it will make this section

of SENS more enjoyable.  The options in the FUNCTION SENSITIVITY MENU

are discussed below.

3.3.7.1  Modify Parameters Option

This  option  allows  the  user  to  modify  the maximum ordinate and

scaling parameters associated with each plot.  This  is  useful  when

trying  to fit several plots on a single graph and either the scaling

or maximium ordinate is off on one or more of the plots.

3.3.7.2  Create a Graph Option

This option allows the user to put one  to  five  (1-5)  plots  on  a

single graph.  The maximum ordinate for the graph will be that of the

maximum  ordinate  associated with the first plot selected.  The user

may select any combinations of real and  imaginary  sensitivities  in

conjuction with the magnitude and phase characteristics.  In general,

the  real sensitivities and magnitude plots mix well together,  while

the imaginary sensitivities and phase plots mix well together.

3.3.7.3  Automatic Graph Generation Option

This option will permit SENS to construct the graphs.  It will do so

by generating the magnitude plot, the phase plot, and then the plots

for the real and imaginary plots with respect to each element.  Note

that SENS will put only one plot on each graph.

3.3.7.4  Display Previous Graph

This option is useful after the user has selected a group of plots to

graph, found that they didn't fit too well, and thus has changed some

of the plotting parameters (option 1).  This option will display  the

last requested set of plots,  but with the new changes.  This way the

user does not have to step  through  option  2  again.

3.3.7.5  Exit Option

Selecting   this   option  will  take  the  user  back  to  the  main

SENSITIVITIES OPTIONS MENU.  Thus, we will return there also.

3.3.8  Modify Element Option

This option will allow the user to go back and  change  an  element's

value.  This  is  helpful  is  the  user  made a typo in the original

value.  It also allows the user to create multiple graphs of function

sensitivity with respect to  an  element  with  the  element's  value

changed in each graph.

3.3.9 Exit Option

Selecting this option will terminate the program.

3.4  LIMITATIONS OF SENS

The following is a summary of  limitations  and  restrictions  within

SENS.

1: SENS  can  handle  a  maximum  of  a tenth order numerator and

denominator polynomials.

2: The  maximum  number of elements and constants declarations is

twenty for each.  An element cannot  be  symbolized  with  the

letters  'CO'  (and  any  combination of case), and a constant

cannot be symbolized with the letter 'N' or 'n' as  the  first

letter.

3: The equation used in SENS to  find  the  root  sensitivity  is

valid  only  for  simple root expressions.  If a multiple root

expression is examined  by SENS, an "infinite" message will be

printed for the unnormalized root sensitivity.   SENS does not

compute  the  normalized  root sensitivity if the value of the

real or imaginary part of the root being analyzed is less than

10E-6.  This is to avoid divide by zero errors.

4: Maximum  number  of  plots per graph is five and the number of

points that can be plotted per graph is 100, due to the nature

of the array used in the plotting routines.

5: SENS  will  only compute the Q and Wn sensitivities for second

and third degree denominator polynomials.