1.0  INTRODUCTION TO ARCGEN 1.01

2.0  HARDWARE/SOFTWARE REQUIREMENTS

3.0  CAPABILITIES

3.1   Amplifier Models

3.2   Circuits Available

3.3   Maximum Order

3.4   Net-List Options

4.0  GETTING STARTED

5.0  ORIENTATION

5.1   Desktop

5.2   Utilities Menu

5.3   Project Menu

5.4   Synthesis menu

5.5   Editor Menu

5.6   Window Menu

6.0  TUTORIAL

6.1   Default Lowpass Filter: Getting Started example

6.2   Narrowband and Broadband Bandpass Filter example

6.3   Simultaneous Comparison between Approximations and Functions example

6.4   Simultaneous Single Function/Multiple Order example

6.5   Single Design: Impedance, Load, and Gain-Bandwidth Effect

6.6   Single Design: Component Precision, Sensitivity, Worst Case Analysis

7.0  CREDITS/BUGS/REVS

1.0   INTRODUCTION

ARCGEN is an Active RC filter synthesis program capable of generating

SPICE format net lists for single and multiple amplifier cascade

analog filters.  The program is dependent on external programs for

circuit simulation, waveform display, and editing of files.  Its target

audience is primarily electrical engineering students and is designed to

provide an introduction to computer aided filter design and especially

the effects of non-ideal characteristics.

The program is menu driven, providing the user a variety of choices

for both filter approximation and synthesis.  Approximation types

available are Butterworth and Chebychev.  Filter function can

be lowpass, highpass, or bandpass.  Additionally, two types of

circuit configurations are selectable: Sallen-Key and Tow-Thomas.

Both can have either ideal or non-ideal active elements.

Two ascii text files are generated: the approximation data/net-list

document <projectname.cir>, and a filter schematic <projectname.sch>.

These are in addition to the files generated by the circuit simulator

(such as output and data).

A user modifiable help utility is accessable during program operation

named ARCGEN.HLP.  Currently, the file contains maximum filter orders,

circuits available, and descriptions of ARCGEN commands.

ARCGEN was developed using the PSpice 5.1 student evaluation version.

An early attempt was made to use MICRO-CAP IV, as it also accepts SPICE

format net-lists.  Slight differences in syntax exist, however, between

PSpice and what MICRO-CAP IV requires (no .PROBE, .MC, etc).  Due to time

restraints these differences were not fully explored in ARCGEN.  Other

simulators have not been attempted.

2.0   HARDWARE/SOFTWARE REQUIREMNTS

*HARDWARE*

IBM PC or compatible, 808X (and above)

Math chip optional

320K base memory available (more may be required by simulator)

Hard disc drive (with some free space)

Monochrome, Hercules, or color monitor (auto-determining)

Graphics capability not necessary (unless required for external

programs)

Mouse optional but makes life easier

Works with "stacked" hard drives

*SOFTWARE*

DOS operating system or compatable virtual - tested with MS-DOS 5.0

Circuit simulator - tested with PSpice 5.1 student evaluation version

Waveform Analyzer - tested with PROBE for PSpice 5.1

ASCII text editor - Such as MS-DOS edit.com program

Note: Problems arose in memory management when running PSpice as a DOS

application under Windows 3.0, after spawning to DOS from ARCGEN.

3.0   CAPABILTIES

3.1   Amplifier Models

Two amplifier models are available.  The first is an ideal amplifier

(VCVS) with a default gain value of 1e+6 V/V (120 dB).  The second

amplifier is a single pole operational amplifier model with user

selectable dc open-loop gain and cutoff frequency.  By default, the

gain is set to 50,000 V/V (94 dB) with a simple pole at 10 Hz.  Input

resistance of the amp is set at 1 Mohm while output is 80 ohms.  A

model was created instead of using the default uA741 (in the PSpice 5.1

device library) in order to maximize filter order.  However, pin-out

for the ARCGEN model is identical to the 741 and similar Op-Amps.

Rails have also been included but are not actively used in the ARCGEN

model; they are simply applied across a high resistance load.  If a

more complex model is required, a suitable editor could be used to cut

and paste the desired model into the net-list.

3.2   Circuits Available

Two filter circuits are available: the Sallen-Key and Tow-Thomas

(feedforward).  The schematics for the former are unique to the filter

function, whereas for the Tow-Thomas the component placement is

identical for all functions (to do this, the necessary resistor or cap

is shorted (value = 9.99e-99) or opened (value = 9.99e+99)).  Gain

is user selectable for the Tow-Thomas but not currently for the

Sallen & Key circuit configurations.  These two configurations are

given as follows:

S A L L E N - K E Y   ( L O W - P A S S  E X A M P L E)

____________C 1________________

|                               |

+ _______R 1____|_____R 2____________|\  X 1    |

|      +|  \_______|_________+

|     __|  /       |

|    | -|/         |

Vin                          C 2   |_______R 4___|       Vout

|    |

|   R 3

- ___________________________|____|_______________________-

T O W - T H O M A S   ( G E N E R I C )

______R 7___________________________________________________

|                                                            |

|______R 8_______Vout                                        |

|              |                                             |

|______C 3_____|        ______C 4_____         ______R 6_____|

|              |       |              |       |              |

|_______|\     |__R 4__|_______|\     |__R 5__|_______|\     |

|      -|  \___|       |      -|  \___|       |      -|  \___|

C 2   ___|  /          R 2   ___|  /          R 3   ___|  /

|   |  +|/   X 2       |   |  +|/   X 3       |   |  +|/   X 4

|  GND                 |  GND                 |  GND

Vin_|______________________|______________________|

3.3   Maximum Order

The maximum order for all filters is 10th order.  "Uni-Pole" refers to

the single-pole amplifier model.  When using PSpice student evaluation

version 5.1 the following are obtainable (due to node count

constraints):

Function    Circuit      Amplifier     Maximum Order

--------    -------      ---------     -------------

Lowpass     Sallen-Key   Ideal             10

Lowpass     Sallen-Key   Uni-Pole          10

Lowpass     Tow-Thomas   Ideal              7

Lowpass     Tow-Thomas   Uni-Pole           5

Highpass    Sallen-Key   Ideal             10

Highpass    Sallen-Key   Uni-Pole          10

Highpass    Tow-Thomas   Ideal              7

Highpass    Tow-Thomas   Uni-Pole           5

Bandpass    Sallen-Key   Ideal             10

Bandpass    Sallen-Key   Uni-Pole          10

Bandpass    Tow-Thomas   Ideal              6

Bandpass    Tow-Thomas   Uni-Pole           4

3.4   Net-List Options

Options available in net-list creation include Monte-Carlo analysis

and Sensitivity/Worst-Case analysis, as resistor/capacitor models are

used on all but the amplifier subcircuits.  Other features available

are discussed in the Synthesis Menu (see subsection 5.4).

4.0   GETTING STARTED

ARCGEN requires use of external circuit simulator, editor,

and waveform display programs.  There are two methods to tell the

program where these are located on the hard drive.  The first is

to include their respective directory location(s) in the PATH=

statement in your autoexec.bat file.  By default the program will look

for a circuit simulator named "pspice1", a waveform analyzer named

"probe", and an editor called "edit".  It is possible to rename an

executable file as long as the program function is compatable.  The

second method is, upon running ARCGEN, to specify the program name

and location in the setup utility (see Utilities Menu, subsection 5.2).

5.0   ORIENTATION

To run ARCGEN, type in "ARCGEN" at the DOS prompt.  A character based

desktop (the main menu), will appear along with a initial pop-up dialog.

The following subsections are descriptions of the desktop and

associated submenus.

5.1   Desktop

The desktop consists of a menubar at the top which is divided into

major headings: Utility, Project, Synthesis, Editor, and Window.  Any

one of these menus can be selected either by clicking with a mouse or

by using the ALT+<highlighted letter>.  At the bottom of the desktop

is a status line.  Depending on program area, various hints, etc. will

appear in it.  By default, three functions are accessable in the

status line:

Menu - "HOME" key               > enables top menubar

Exit - "ALT+X" key combination  > brings up "exit to DOS?" prompt

HELP - "F10" key                > loads and displays ARCGEN.HLP

You can either click on the function directly or enter the keyboard

command as indicated.

NOTE: All menu selection titles not followed by "..." will execute the

function immediately, otherwise an intermediate step occurs,

allowing the user a chance to reconsider.

5.2   Utilities Menu

The Utilities selections consist of:

* Setup...                        ALT+T

When selected, a dialog box pops up prompting the user for

programs/paths of the three necessary executable programs:

editor, circuit simulator, and waveform analyzer.  Path names

are optional (but a slight decrease in run time will result if

included).  By default, ARCGEN looks for programs named "edit",

"pspice1", and "probe".  These may be changed to meet user

requirements at any time during program operation. Additionally,

an option is provided whether or not to create approximation data and

update the desktop. By not doing so much time is saved. By default

this option is not selected.

* Current Directory...            ALT+C

This utility provides the user a means to change the current

(active) directory/drive to any available.  It is equivalent to

the DOS command "cd" or "X:".  As a note, when program operation is

terminated, the selected directory will be the one active upon exit.

* DOS Spawn                       ALT+D

DOS spawn is an immediately executed command.  Unlike the typical

DOS shell, an image is made of the current program state (about 1.8k

bytes), memory is deallocated, and the user enters DOS with a prompt of

"$".  One word of caution though: do not attempt to load TSRs while in

DOS as memory will be fragmented leading to unpredictable results.  To

return to ARCGEN, type in "EXIT".

* Exit...                         ALT+X

By selecting this command, a dialog box will appear prompting the user

whether to exit to DOS or not.  By leaving the program, all filter

specification and operation variables will be lost.  If entering DOS is

necessary, take the DOS Spawn route to preserve entered data unless, as

mentioned earlier, TSR's must be loaded.

5.3   Project Menu

Every time a filter is designed it must be associated with a unique

project name.  All corresponding files and activities will have this

name.  There are two ways to initialize a project:

* Overwrite Existing...           ALT+O

When selected, a pop-up dialog will appear which by default shows all

files in the current directory having a ".cir" extension.  If none are

shown, you may go up and down the directory hierarchy until the desired

file is found, but cannot load it.  To access the desired file, use the

Utilities/Current Directory selection first.  Remember... once a file

is loaded, the initial write to the file will overwrite what's already

there (not append), unless write protected.

* Create New...                   ALT+N

By selecting this, you will have two options.  The first is to specify

a project name, and the second is allow ARCGEN to.  If the latter is

opted, the ARCGEN will automatically name the project "nonamexx", where

"xx" is a number ranging from 00-99.  If a file already exists (ie.,

noname00) the number will be incremented until no files exist with that

name.  The extension ".cir" is always concatenated to the end of the

project name, whether an extension is specified or not.

NOTE: When either of these selections are executed, the program

automatically will bring up the "Approximate Function...",

submenu of the Synthesis menu (see subsection 5.4).  This seemed

a logical progression.

5.4   Synthesis Menu

These selections are the 'meat' of the program:

* Approximate Function...         ALT+1

When selected a pop-up menu appears prompting the user to select the

filter function (lowpass, highpass, bandpass) and approximation type

(Butterworth, Chebychev).  Another function available is for order

comparison between Butterworth, Chebychev, Inverse Chebychev, and

Elliptic approximations, as pertaining to maximum passband deviation

and minimum stopband attenuation.  This option is denoted "ORDER

COMPARISON" and has no limit in filter order.  Time restraints prevented

inclusion of Inverse Chebychev and Elliptic filter appoximations but

order comparison for these was retained for information puposes.

Another option is "[ ] Append".  This allows multiple approximations

to be written to the same file.  It is useful for comparing either

filter approximation types or functions. As a note: simulation

frequency sweep range must be entered manually with this option.

Additionally, PROBE will not autodisplay the waveform.  This was

necessary as there are too many permutations to deal with effectively.

The user has the option of 'CANCEL'ling (not accepting any changes)

and returning to the main menu, reverting selections to 'DEFAULT' and

reloading the menu, or accepting the selections and continuing on in

the 'APPROX'imation.

If 'APPROX'imation is selected a second pop-up dialog appears which

prompts for filter specifications.  The areas of concern are:

[Kp/Ripple]     > (Butterworth) Maximum passband deviation

(Chebychev) Maximum passband ripple

[Ks]            > Minimum stopband attenuation

[wp]            > Passband 3-dB frequency (at which Kp is met)

[BWp]           > (bandpass) Passband width

[ws]            > Stopband frequency (at which Ks is met)

[BWs]           > (bandpass) stopband width

[s]             > Frequency multiplier (denormalization)

[H]             > Passband gain (realizable only in Tow-Thomas

lowpass, highpass, and bandpass)

(Radians)> Specifications entered in radian mode

(Hertz)  > Specifications entered in hertz mode

NOTE: For bandpass approximations, [BWp] determines the passband width.

For anything different than [BWp] = 1 radian (0.1592 Hz.), a

frequency denormalization is applied first to the lowpass

approximation then bandpass transform occurs.  For narrowband

approximation, specify [BWp] < 1 radian (0.1592 Hz.).  For

broadband approximation, specify [BWp] > 1 radian (0.1592 Hz.).

Variable [s] is always applied after the lowpass to bandpass

transformation, shifting (multiplying) the entire function by [s].

See example 6.2 for narrowband and broadband examples.

If all is acceptable, the user can select the "SYNTHESIZE" button

to continue on.  If not, "DEFAULT" can be chosen to revert all values

and options to default (and automatically repeat the dialog).  Another

selection, "VIEW" is available which enables the approximation data to

be generated, and presented at the desktop. The program will then wait

for subsequent input at the main menu.

* Create Net List...              ALT+2

This command produces a pop-up dialog to be generated which allows the

user to choose options and values as pertaining to the net-list to

be generated.  A breakdown of these options are as follows:

---------------------------------------------------------------------

[Use uni-Pole OP-Amps]  : If selected, the single pole Op-Amp model

will be used as the active circuit element.

If not selected an 'ideal' VCVS is used,

which is default.

[Append Circuit]        : This option enables multiple net-lists to be

generated in one file <projectname.cir>.  Its

use is to compare two or more circuits using

the waveform analyzer.  By default it is not

selected.

[Include .PROBE]        : This option will write a probe statement to

the net-list, for graphical analysis of the

filter data.  It is selected by default.

[Include .PRINT]        : This option will write a "print" statement

to the net-list, for tabular output data. It

is not selected by default.

[Include .PLOT]         : This option will write a "plot" statement to

the net-list, causing a character based plot

to be generated when running the simulator.

This option is not selected by default.

[Monte Carlo]           : This option enables the Monte Carlo analysis

capability of PSPICE.  In conjunction with

this selection, the number of Monte Carlo

runs (minimum 2), and component tolerances

can be specified.  Basically, Monte Carlo

varies the component tolerances randomly

with each run.  This option is not selected

by default.

[Sensitivity/Worst Case]: This option enables the Sensitivity & Worst

case analysis capability of PSPICE. In

conjuntion with this selection, component

tolerances can be specified.  Sensitivity

analysis varies each component by the

specified tolerance (+).  Worst case is then

run which varies all components (except the

ones in the amplifier) simultaneously at

their value+tolerance values.  Data is stored

for subsequent analysis (such as PROBE).

This option is not selected by default.

[Decade Sweep]          : By selecting this option, simulation occurs

on a logarithmic sweep.  If not selected

a linear sweep will be used.  By default

the Decade Sweep is used.

[Voltage Transfer Only] : By default, the simultor is instructed only

to perform analysis at the output node.  If

this option is not selected, an analysis

will be made of all circuit nodes, increasing

run time.

[Analyzer Autorun]      : This option will automatically load and run

the program specified in the Setup dialog

denoted as "Waveform Analyzer".  It is

selected by default. As a note, the chosen

simulator will be run whether this option

is selected or not.

[Auto Display]          : If the waveform analyzer is "autorun

selected" (previous option), a command file

will be generated which automatically causes

the output cicuit node waveform to be

displayed.  If multiple output node data is

generated, all resultant waveforms will be

displayed.  This option is selected by

default.

Sample Points           : This numerical input field specifies the

number of sample points per decade/linear

range to be performed during circuit

simulation.  Usually, for higher order

filters with varying passband deviation (such

as the Chebychev), the number of sample points

should be increased.  By default the number

of sample points is 50.

Start Sweep             : This numerical input field accepts values

in Hertz, specifying the start frequency

of circuit simulation.  By default it is

zero hertz.  If left as such, and the next

field (Stop Sweep) is also zero, ARCGEN will

automatically range the start/stop frequency

depending on the filter function (lowpass,

etc.).

Stop Sweep              : Also in Hertz - see "Start Sweep".

Monte Carlo Runs        : This numeric input field (default is zero),

specifies the number of Monte Carlo runs

when the Monte Carlo option is selected.

A minimum of two is necessary for Monte

Carlo analysis.

Resistor Tolerance %    : This numeric input field specifies the

tolerance (positive) to be associated with

all resistors for use in Monte Carlo or

Sensitivity/Worst Case analysis.  Components

within the amplifier model are not affected

by this tolerance.

Capacitor Tolerance %   : This numeric input field specifies the

tolerance (positive side) to be associated

with all capacitors for use in Monte Carlo or

Sensitivity/Worst Case analysis.  Components

within the amplifier model are not affected

by this tolerance.

Precision               : This numeric input field will limit the

generated net-list component fractional

value to that specified.  By default it is

set to five decimal places. This function

is useful in limiting components to

realizable values, ie. 2 - 2 places.

(Sallen & Key Cascade)  : Selected as default, this option will cause

a Sallen & Key filter circuit to be used.

(Tow-Thomas Cascade)    : If selected, the Tow-Thomas filter circuit

will be generated.  As a note either this

or the Sallen & key can be used at one time.

Hold Resistors         : This numeric input field effects the

impedance denormalization as it will hold all

independent resistors to the user specified

value.  Sometimes the component spread can be

minimized by this instead of using "Hold

Capacitors...".  By default the value is

zero (do not hold resistors).

Hold Capacitors         : This is analogous to the "Set Resistors:"

field.  Likewise with component spread.

By default the value is 1 Farad (use).

R load                  : This numeric input field is used only when a

resistive load is desired at the circuit

output.  It is specified in kilo-ohms and

by default is zero (no load).

C load                  : This numeric input field is used only when a

capacitive load is desired at the circuit

output.  It is specified in microFarads and

by default is zero (no load).

DC Gain, V/V            : This numeric input field specifies the open

loop gain of the non-ideal Op-Amp model.

It is not used for the ideal VCVS model.  By

default the value is 50,000 V/V (94 dB).

Bandwidth, Hz           : This numeric input field works in conjunction

with the "DC Gain" field, specifying the open

loop bandwidth of the non-ideal Op-Amp model.

By default its value is 10 Hz.

----------------------------------------------------------------------

When all values and options have been specified and selected, the user

has four commands available.  The first is to accept the state of the

dialog box and continue on to "SIMULATE".  By "simulate" it is meant

that the circuit simulator will then be called upon to perform.  The

second command option is to "VIEW".  By selecting this, the netlist

will be generated and displayed (if enabled) with the program then

awaiting further user action at the main menu.

NOTE: Unless "[ ]  Append" is selected, the approximation data for a

filter is only generated once.  The initial net-list write will

append to the approximation data. Subsequent net-list writes,

unless "[ ] Append Circuit" is selected, will overwrite the

approximation data.

Another command option is "DEFAULT", which restores all values and

options to default.  The dialog is restored to default awaiting

user action.  The last command option is "CANCEL".  By selecting, no

changes (in the active dialog box) made will be stored.  The

program will then return to the main menu.

* Simulate Circuit                ALT+3

This command runs the circuit simulator. This selection is only

available if the current net-list was generated by the "Create Net

List..." command in the current session.  It is an immediate command.

* Waveform Analyzer               ALT+4

This command runs the waveform analyzer. This selection is only

available if the current net-list was generated by the "Create Net

List..." command, run through the circuit simulator with, in this

case ".PROBE", opted in the current session.  It also is an immediate

command.  As a note, data point capability is quite large, although

(on my PC) it ran out of memory when running a 10th order Tow-Thomas

in Sensitivity/Worst Case analysis.

* Generate Schematic              ALT+5

This command is available immediately after the "Create Net List..."

command has been executed (whether by the "simulate" or "View" command

options).  You'll see your hard drive light cycle on and off while

each filter stage is written to the <project_name>.SCH file in the

current directory.  A short beep will then be generated to indicate

completion.

5.5   Editor Menu

The editor menu provides the user with means to manually edit or view

the generated net-list or schematic.  By accessing the editor, the

files can then be printed or, in the case of the net-list, resubmitted

to the simulator.  The current state of the program is retained while

using the editor, with the net-list reloading for display upon leaving

the editor.

* Edit Net List                   ALT+6

This selection will run the editor and load the current net-list.

Once the editor is left, the program will reload the net-list.

* Edit Schematic                  ALT+7

This selection will run the editor and load the current schematic.

5.6   Window Menu

* Close Project                   DELETE

This selection was added to enable non-mouse users a ready means to

close projects.  By doing so, the current state of program data will

be set to default and only the "Setup" and "Project" menus will be

accessable (as they are during program start).

6.0   EXAMPLES

These examples are provided to exercise most program functions,

develop user familiarity, and to display what the program is capable of.

6.1   Default Lowpass Filter: Getting Started

This example produces a 10th order Butterworth lowpass filter. Also,

the schematic generation utility and editor access are utilized.

After the program has been loaded: Press "Enter" at the keyboard, or

click on [OK] on the displayed dialog box.

Select    [Project] from the main menu

Select    [Create New...]

- A dialog box now appears prompting for a project name

Press     ENTER at the keyboard (or click on [OPEN] at the dialog)

- A new project desktop will appear named "noname00.cir".

(If "noname01", "noname02",... is displayed, a lower numbered file

already exists.)

- Since a new project has been loaded, the "Filter Select" menu

automatically appears.

select    [APPROX]

select    [SYNTHESIZE]

select    [SIMULATE]

- Waveform is displayed with amplitude of 11.125V. The circuit and

design used is the Sallen & Key equal R/equal C.

Exit PSpice

Select    [Synthesis] from the main menu

select    [Generate Schematic]

- A "beep" will sound when the schematic generation is complete

select    [Editor] from the main menu

select    [Edit Schematic]

- Schematic displayed

Exit editor

select    [Editor] from the main menu

select    [Edit Netlist]

Using the editor, change the Net List line reading (page down to right

after "* Circuit Stimulus"):

"Vin      5       0       AC      1.0V"

to

"Vin      5       0       AC      100.0V"

Exit the editor (make sure to save the file first)

select    [Synthesis] from the main menu

select    [Simulate Circuit]

- Waveform displayed with amplitude now 100 times first waveform

Exit PSpice

****** END OF EXAMPLE ******

6.2   Narrowband and Broadband Bandpass Filters

This example will produce, via two separate runs, two Butterworth

bandpass filters.  One will be a narrowband filter, while the

other a broadband.  The effects of varying pass bandwidth, stop

bandwidth, and stop band attenuation are shown.

Starting at the "Filter Select" menu (pops-up when project loaded or

by selecting [Synthesis] from the main menu, then

[Approximate Function...]).

*** Narrowband Example ***

select    (*) Bandpass

select    [APPROX]

enter        12   for  [Ks]

enter        .1   for  [BWp]

enter        .2   for  [BWs]

select    [SYNTHESIZE]

select    [SIMULATE]

- Waveform displayed : The displayed waveform has a center frequency of

1 rad/s, pass bandwidth of .1 rad/s, and stop

bandwidth of .2 rad/s. Notice that the magnitude

is nearly 761V (for a 1V input) using this

design.

Exit PSpice

*** Broadband Example ***

select    [Synthesis] from the main menu

select    [Approximate Function...]

select    [APPROX]

enter         5   for  [BWp]

enter        10   for  [BWs]

select    [SYNTHESIZE]

select    [SIMULATE]

- Waveform displayed : The displayed waveform still has a center

frequency of 1 rad/s, pass-bandwidth of 5 rad/s,

stop-bandwidth of 10 rad/s, but now only has an

amplitude of about 87mV.

Exit PSpice

****** END OF EXAMPLE ******

6.3   Simultaneous Comparison between Approximations and Functions

This example provides six simultaneous waveforms:

1. Butterworth lowpass (3rd order)

2. Butterworth highpass (3rd order)

3. Butterworth bandpass (6th order)

4. Chebychev lowpass (3rd order)

5. Chebychev highpass (3rd order)

6. Chebychev bandpass (6th order)

NOTE: Since this is rather a long analysis by PSpice, the data file

has been provided under file name EX6_3T_T.DAT.  To view the

waveforms: At the dos prompt (either spawn from ARCGEN or run

from DOS), type PROBE <filename>.

NOTE: By selecting "Do not update..." from the UTILITIES/SETUP menu,

run time will be greatly decreased.  For the remaining examples

this is recommended.

Starting at the "Filter Select" menu (pops-up when project loaded or

by selecting [Synthesis] from the main menu, then

[Approximate Function...]).

select    [APPROX]

enter        17   for [Ks]

select    [SYNTHESIZE]

A "beep" will be heard if "Do not update..." is enabled

select    (*)     Tow-Thomas

enter     .0016   for "Start Sweep"

enter        16   for "Stop Sweep"

select    [VIEW]

A "beep" will be heard if "Do not update..." is enabled

select    [Synthesis] from the main menu

select    [Approximate Function...]

select    [X]     Append

select    (*)     Highpass

select    [APPROX]

select    [SYNTHESIZE]

A "beep" will be heard if "Do not update..." is enabled

select    [*]     Append Circuit

select    [VIEW]

A "beep" will be heard if "Do not update..." is enabled

select    [Synthesis] from the main menu

select    [Approximate Function...]

select    (*)     Bandpass

select    [APPROX]

select    [SYNTHESIZE]

A "beep" will be heard if "Do not update..." is enabled

select    [VIEW]

A "beep" will be heard if "Do not update..." is enabled

select    [Synthesis] from the main menu

select    [Approximate Function...]

select    (*)     Chebychev

select    [APPROX]

select    [SYNTHESIZE]

A "beep" will be heard if "Do not update..." is enabled

select    [VIEW]

A "beep" will be heard if "Do not update..." is enabled

select    [Synthesis] from the main menu

select    [Approximate Function...]

select    (*)     Highpass

select    [APPROX]

select    [SYNTHESIZE]

A "beep" will be heard if "Do not update..." is enabled

select    [VIEW]

A "beep" will be heard if "Do not update..." is enabled

select    [Synthesis] from the main menu

select    [Approximate Function...]

select    (*)     Lowpass

select    [APPROX]

select    [SYNTHESIZE]

A "beep" will be heard if "Do not update..." is enabled

select    [SIMULATE]

The program will now run each netlist in order, with all

run under one name.

At PROBE:

select    "All_AC_sweep"

You will now get a rather lengthy message, prompting whether to

"Skip_inconsistant_sections" or "Do_not_skip_sections".  The latter

one is the one you want. These options were presented as the two

circuit types do do have the same number of nodes, thus nodes could

have two meanings.

select    "Do_not_skip_sections"

select    "Add_trace"

enter     v(11)

select    "Add_trace"

enter     v(23)

- waveforms displayed

*** END OF EXAMPLE ***

6.4   Simultaneous Single Function/Multiple Order

This example produces seven waveforms, corresponding to order.

The approximation used is a Chebychev in a lowpass function.

Order ranges from one to seven.

NOTE: Since this is rather a long analysis by PSpice, the data file

has been provided under file name EX6_4T_T.DAT.  To view the

waveforms: At the dos prompt (either spawn from ARCGEN or run

from DOS), type PROBE <filename>.

To decrease time, select the "Do not update desktop..." option in the

"Setup" menu.

Starting at the "Filter Select" menu (pops-up when project loaded or

by selecting [Synthesis] from the main menu, then

[Approximate Function...]).

select    (*)     Order Comparison

select    [APPROX]

enter       1     for [Kp]

enter       3     for [Ks]

select    [VIEW]

note that to retain the current order (1st), [Ks]max can be 3.08709

select    [OK]

select    [APPROX]

enter       5     for [Ks]

select    [VIEW]

note that to retain the current order (2nd), [Ks]max can be 11.3631

select    [OK]

select    [APPROX]

enter       15    for [Ks]

select    [VIEW]

note that to retain the current order (3rd), [Ks]max can be 22.4558

select    [OK]

select    [APPROX]

enter       25    for [Ks]

select    [VIEW]

note that to retain the current order (4th), [Ks]max can be 33.8688

select    [OK]

select    [APPROX]

enter       35    for [Ks]

select    [VIEW]

note that to retain the current order (5th), [Ks]max can be 45.3059

select    [OK]

select    [APPROX]

enter       50    for [Ks]

select    [VIEW]

note the to retain the current order (6th), [Ks]max can be 56.7447

Since this is as far as we're going, we'll stop here at 7th order

by entering "65" when needed.

select    [OK]

select    (*)     Chebychev

select    [APPROX]

enter       3     for [Ks]

select    [SYNTHESIZE]

A "beep" will be heard if "Do not update..." is enabled

enter     (*)     Tow-Thomas

enter     .001    for "Start Sweep"

enter       1     for "Stop Sweep"

select    [VIEW]

A "beep" will be heard if "Do not update..." is enabled

select    [Synthesis] from main menu

select    [Approximate Function...]

select    [X]     Append

select    [APPROX]

enter       5     for [Ks]

select    [SYNTHESIZE]

A "beep" will be heard if "Do not update..." is enabled

select    [X]     Append Circuit

select    [VIEW]

A "beep" will be heard if "Do not update..." is enabled

select    [Synthesis] from main menu

select    [Approximate Function...]

select    [APPROX]

enter       15    for [Ks]

select    [SYNTHESIZE]

A "beep" will be heard if "Do not update..." is enabled

select    [VIEW]

A "beep" will be heard if "Do not update..." is enabled

select    [Synthesis] from main menu

select    [Approximate Function...]

select    [APPROX]

enter       25    for [Ks]

select    [SYNTHESIZE]

A "beep" will be heard if "Do not update..." is enabled

select    [VIEW]

A "beep" will be heard if "Do not update..." is enabled

select    [Synthesis] from main menu

select    [Approximate Function...]

select    [APPROX]

enter       35    for [Ks]

select    [SYNTHESIZE]

A "beep" will be heard if "Do not update..." is enabled

select    [VIEW]

A "beep" will be heard if "Do not update..." is enabled

select    [Synthesis] from main menu

select    [Approximate Function...]

select    [APPROX]

enter       50    for [Ks]

select    [SYNTHESIZE]

A "beep" will be heard if "Do not update..." is enabled

select    [VIEW]

A "beep" will be heard if "Do not update..." is enabled

select    [Synthesis] from main menu

select    [Approximate Function...]

select    [APPROX]

enter       65    for [Ks]

select    [SYNTHESIZE]

A "beep" will be heard if "Do not update..." is enabled

select    [SIMULATE]

The program will now run each netlist in order, with the entire

run under one name.

At PROBE:

select    "All_AC_sweep"

You will now get a rather lengthy message, prompting whether to

"Skip_inconsistant_sections" or "Do_not_skip_sections".  The latter

one is the one you want. These options were presented as the two

circuit types do do have the same number of nodes, thus nodes could

have two meanings.

select    "Do_not_skip_sections"

select    "Add_trace"

enter     v(5)

select    "Add_trace"

enter     v(11)

select    "Add_trace"

enter     v(17)

select    "Add_trace"

enter     v(23)

- waveforms displayed

*** END OF EXAMPLE ***

6.5   Single Design: Impedance, Load, and Gain-Bandwidth-Product Effects

This example explores parameter variations on a single design, using

a Chebychev 4th order bandpass Sallen & Key filter.

NOTE: Since this is quite a long analysis by PSpice, the data files

for both the Sallen & Key and Tow-Thomas configurations have

been provided under file names EX6_5S&K.DAT and EX6_5T_T.DAT

respectively.  To view the waveforms: At the dos prompt (either

spawn from ARCGEN or run from DOS), type PROBE <filename>.

To decrease time, select the "Do not update desktop..." option in the

"Setup" menu.

Starting at the "Filter Select" menu (pops-up when project loaded or

by selecting [Synthesis] from the main menu, then

[Approximate Function...]).

select    (*)  Bandpass

select    (*)  Chebychev

select    [APPROX]

enter        16   for  [Ks]

enter     62830   for  [s]

select    [SYNTHESIZE]

A "beep" will be heard if "Do not update..." is enabled

enter       100   for "Sample Points"

select    [SIMULATE]

- Waveform displayed : As ideal, this waveform serves as the basis for

comparison.  As can be measured using the

PROBE "Cursor" function, the peaks occur at

(approx.) 12.79V/V (22.124dB).  Gain is not

user specifiable with this Sallen & Key

design; if the Tow-Thomas circuit is used

(in which gain is user specifiable) be sure

to use this value for [H] Gain (dB).

Exit PSpice

select    [Synthesis] from the main menu

select    [Create Net List...]

select    [X]  Use Uni-Pole Op-Amps

select    [X]  Append Circuit

select    [SIMULATE]

- Waveform displayed : As can be seen, the result is highly distorted.

Exit PSpice

select    [Synthesis] from the main menu

select    [Create Net List...]

deselect  [ ]  Use Uni-Pole Op-Amps

enter     .001    for "Hold Resistors"

enter     0       for "Hold Capacitors"

select    [SIMULATE]

- Waveform displayed : As can be seen, the result is now ideal.

Exit PSpice

select    [Synthesis]  from the main menu

select    [Create Net List...]

select    [X]  Use Uni-Pole Op-Amps

select    [SIMULATE]

- Waveform displayed : As can be seen, the result is a little better

than when the independent capacitors were held

to 1F.  As can be seen on the netlists, the

component spread is less using this method (all

resistors same order of magnitude).

Exit PSpice

select    [Synthesis]  from the main menu

select    [Create Net List...]

select    [DEFAULT]

enter     10     for "Hold Resistors:"

select    [SIMULATE]

- Waveform displayed : Looking a lot better, however, a slight

magnitude and center frequency problem still

exist.  By specifying a higher frequency and

gain initially specs could be met. As a note

the normalized gain*banwidth product (at a

frequency of 10 Khz) is 50.

Exit PSpice

select    [Synthesis]  from the main menu

select    [Create Net List...]

enter     1000000    for "Bandwidth"

select    [SIMULATE]

- Waveform displayed : Again, back to ideal.

Exit PSpice

select    [Synthesis]  from the main menu

select    [Create Net List...]

enter     50000      for "Bandwidth"

enter     200        for "Gain"

select    [SIMULATE]

- Waveform displayed : As expected, again ideal with the normalized

Gain*Bandwidth product at 1000.

Exit PSpice

select    [Synthesis]  from the main menu

select    [Create Net List...]

enter     .008     for "Hold Resistors:"

enter     .00005   for "Hold Capacitors:"

select    [SIMULATE]

- Waveform displayed : We now find somewhat the same effects as before,

showing how external factors can effect filter

operation.  This load configuration could

simulate an 8 ohm speaker with 5pF stray

capacitance.

Exit PSpice

NOTE: Although all waveforms are shown simultaneously, each one could

have been presented seperately by deselecting "Append Circuit"

at the start of the example.  It is also interesting to use

a Tow-Thomas configuration in this example; quite different

results (but remember to match the gain as Tow-Thomas is control-

able). To save some time: run PSpice only after all the

seperate net lists have been created (such as was done in

example 6.4).

*** END OF EXAMPLE ***

6.6   Single Design: Component Precision, Sensitivity, Worst Case Analysis

This example explores manufacturing concerns with the same single

design used in example 6.5 (Chebychev 4th order bandpass Sallen & Key

filter).

NOTE: Since this is quite a long analysis by PSpice, the data files

for both the Sallen & Key and Tow-Thomas configurations have

been provided under file names EX6_6S&K.DAT and EX6_6T_T.DAT

respectively.  To view the waveforms: At the dos prompt (either

spawn from ARCGEN or run from DOS), type PROBE <filename>.

To decrease time, select the "Do not update desktop..." option in the

"Setup" menu.

Starting at the "Filter Select" menu (pops-up when project loaded or

by selecting [Synthesis] from the main menu, then

[Approximate Function...]).

select    (*)  Bandpass

select    (*)  Chebychev

select    [APPROX]

select    (*)  Hertz

enter       16       for  [Ks]

enter     .1591549   for  [wp]

enter     .31830989  for  [ws]

enter      10000     for  [s]

select    [SYNTHESIZE]

A "beep" will be heard if "Do not update..." is enabled

enter       75   for "Sample Points"

select    [VIEW]

A "beep" will be heard if "Do not update..." is enabled

select    [Synthesis] from main menu

select    [Create Net-List...]

select    [X]     Append Circuit

select    [X]     Sensitivity/Worst Case

enter       5     for "Resistor Tolerance %"

enter      20     for "Capacitor Tolerance %"

enter       1     for "Precision (0.123...)"

select    [SIMULATE]

Now, take a break (or a vacation, depending on your computer)...

- Waveforms Displayed: As is observed; by limiting the component

precision a slight dissymmetry occurs along

with a magnitude increase. By varying each

component by its (+) tolerance singularly

does'nt have much effect, but varying ALL

components simultaneously does.  It really

is'nt too bad though in comparison to the

Tow-Thomas case (EX6_6T_T.DAT).

****** END OF EXAMPLE ******

7.0  CREDITS/BUGS/REVS

T R A D E M A R K   A C K N O W L E D G E M E N T S

1. IBM PC is a registered trademark of International Business Machines

Corporation.

2. Turbo C++ and TurboVision are trademarks or registered trademarks

of Boland International Corporation.

3. Windows 3.X, as referenced in this user guide, refers to Microsoft's

implementation of a windows system.

4. MS-DOS is a trademark or registered trademark of Microsoft

Corporation.

5. PSpice and PROBE are trademarks or registered trademarks of

Microsim Corporation.

6. MICRO-CAP IV is a trademark of Spectrum Software, Inc.

7. General Notice: Some of the product names used herein have been used

for identification purposes only and may be trademarks of their

respective companies.

P U B L I C   D O M A I N   S O F T W A R E   U S E D

1. Borland's TInputLine class was replaced with that created by

Michael Bonner, TField, CIS id 73250,1460

2. Numeric only data entry was implemented in ARCGEN with a modified

version of NumInputLine, created by Jim Price, CIS id 76264,3534.

3. The DOS spawn routine was a modified version of EXEC.C and

associated assembly code (version 2.3), by Thomas Wagner,

BIXname: twagner.

T H A N K S   T O . . .

Thanks must go to Dr. L.P. Huelsman, who as advisor, allowed me

to take the project into my own hands.  The suggestions and

remarks were very helpful; unfortunately all could not be

incorporated.  As a note: most filter terminology used in ARCGEN

originated (for me) from his text "Active and Passive Analog Filter

Design: An Introduction", McGraw Hill, Inc., 1993, with the

Sallen & Key design 1 used as the basis in ARCGEN.  The Tow-thomas

can be found, in closely similar form, in the text "Microelectronic

Circuits", 3rd edition, Sedra/Smith, HRW Saunders, 1991.

F I N A L L Y . . . K N O W N   B U G S

General...

1.  Desktop viewer not expanding tabs.

2.  Have seen two cases of ARCGEN generating "abnormal program

termination" upon exit.  Both times were after long period

of heavy PSpice use.  May be associated with deletion of

command file for probe upon exit.  This may be local to my

clone 286.

3.  Occasional screen write anomally: get momentary all white or

red desktop in time between writing netlist and loading same

netlist.  This may be local to my clone 286.

4.  If a file loaded to the desktop exceeds 64k memory, the error

handler -usually- picks it up.  It did not always when using

this entire file as "arcgen.hlp".

Technical...

1.  Realized gain in the Tow-Thomas Chebychev bandpass case differs

from that specified when using ripple other than 3.0103dB. The

difference is usually not very much though.

R E V I S I O N S

1.00    Pre-release

1.00a   Alpha release

1.01    First release