README.DOC

-----------------

This .DOC file gives a brief description of the functions

given in the header files (*.h), and explains the the format of

the user-supplied main function.

HEAD.H

----------------

This header file includes the standart run-time C libraries, such as,

<stdio.h>, <math.h>, etc., and the definitions generally used by the

following header files and the main program.

MEMALL.H

---------------

This header file has the dynamic memory allocation routines, and a file

opening routine of the specified file name, extension and the mode.

--- void errmsg(char *msg)

Prints out the error message 'msg' and exits.

--- int index(char *s,char c)

Returns the location of first occurence of character 'c' in

string 's'.

---  float *vector(unsigned int u)

Allocates memory at run time of the size 'u' for a floating point

array, array[0..u]. If there is not enough memory, prints out

an error message and exits.

Example Usage:

float *x;

x = vector(5); (allocates memory for x[0..5])

---	int *ivector(unsigned int u)

Integer counterpart of 'vector()'.

--- float **matrix(int rh,unsigned int ch)

Allocates memory for a 2-dimensional floating point array of the

form matrix[0..rh][0..ch]. In case of not enough memory, it prints

out an error message, and exits.

--- void free_vector(float *v)

--- void free_ivector(int *v)

--- void free_matrix(**v, int rh)

Free memory allocated by their respective memory allocation routines.

DIFF.H

--------------

Contains the differential equation solving routines.

--- void minmaxx(float *x,int n, float *minx,float *maxx)

Finds the minimum and maximum of array x[0..n], and return them in

minx, and maxx respectively.

- x: A floating point array of x[0..n]

- n: Upper limit of x

- minx: After run, contains the minimum of x[0..n]

- maxx: After run, contains the maximum of x[0..n]

--- void rk4(float y[],float dydx[],int n,float x, float h,

float yout[],void (*f)(float, float * ,float *))

Fourth order Runge-Kutta method. Given values for n variables y[1..n]

and their derivatives dydx[1..n] known at x, advances the solution over

h (stepsize) and returns the incremented values in yout[1..n]. The user

supplies the function f(x,y,dydx) which returns the derivatives at x.

Example Usage:

See example files.

GTOOLS.H

-------------------------

Contains text input,output and X-Y plotting routines in the graphics mode.

--- int gprintf(char *fmt, ...)

--- int gprintfxy(int x,int y,char *fmt,...)

--- int gscanf(char *fmt,...)

--- int gscanfxy(int x,int y,char *fmt,...)

--- char *ggets(char *buffer)

A collection of graphics based input output routines. They all

assume LEFT_TEXT and HORIZ_DIR justification. For a detailed explanation

see the file GTOOLS.H.

All the routines in this header file assume VGA color monitor

with 640 x 480 resolution. They can be adapted for any type of

monitor with small modifications. The screen is assumed to have

four rectangular areas (0 being the whole screen) as shown below,

each being 240 pixels wide and 200 pixels long.

|------------------------------|

|              0               |

| |----------|  |----------|   |

| |     1    |  |    2     |   |

| |----------|  |----------|   |

|                              |

| |----------|  |----------|   |

| |     3    |  |    4     |   |

| |----------|  |----------|   |

|------------------------------|

--- void grafini()

Initilizes the grapichs mode. Assumes the .BGI files are in the current

directory.

--- void putframew(int xgrid, int ygrid)

Draws a rectangular frame, X and Y axes having xgrid and ygrid number

of grids. Each grid has 5 subdivisions.  Called by putwinw().

--- void putwinw(float xmin,float xmax,float ymin,float ymax,int xgrid,

int ygrid,int region)

Calls putframew() for the region specified by region. Puts numbers

from xmin to xmax and ymin to ymax for X and Y axes respectively at

equal intervals specified by xgrid and ygrid.

--- void plotxy(float *x,float *y, int np,float xmin,float xmax,

float ymin,float ymax,int color,int xgrid,int ygrid,int region)

Calls putwinw() for the region specified by region. Plots an X-Y

graphics (x vs y) with np number of points.

- x: Points for X-axis

- y: Points for Y-axis

- np: Number of points  for x and y to be plotted

- xmin, xmax: Minimum amd maximum values for the X axis

- ymin, ymax: Minimum amd maximum values for the Y axis

- color: Drawing color

- xgrid, ygrid: Number of grids for X and Y axes.

- region: One of the regions (0,1,2,3,4) as shown above.

--- void sscreen(FILE *fp)

Saves the screen to the file pointed to by the file pointer fp.

--- void rscreen(FILE *fp)

Restores the screen from the file pointed to by the file pointer fp.

--- FILE *gopen(char *fname, char *mode, char *ext)

Opens a file by the specified filename "fname.ext" in the mode "mode" (i.e.,

read, write, etc..) and returns a file pointer to the opened file.

Example Usage:

FILE *fp;

fp = gopen("ali","w","dat")

opens the file names "ali.dat" for writing.

For usage see the example files.

FOUR.H

----------------------

Contains a routine for the FFT (fast fourier transform)

--- void four(float d[],int nn,int isign)

Replaces d by its discrete fourier Xr. nn must be an integer power of 2.

isign determines the direction of the transformation. See "four.h" file

for a better explanation.

--- void fourout(float *d,float *x,float *y,int n,float dt)

Arranges data returned by four() for plotting purposes.

- d : Input as returned by four1()

- x : frequency points, x[0..1/(2*dt)]

- y : magnitude squared (y[i] = d[2*i]*d[2*i] + d[2*i+1]*d[2*i+1])

- n : Same as nn above

- dt : Time intervals in which the original data is taken

See example files.